6-16-02

Tested Ed’s radiator, and my first attempt at a flat rad.  The test setup was as follows, hot water was run from the sink faucet, through a garden hose, to a ½” ID tube, through the rad.  The rad was submerged in the pool, about 5 feet from the outlet near the steps.  There is a thermocouple in the ½” ID tube going into the rad, and out of the rad.

Ed’s Rad

Overall dimensions- 13.5” x 9” x 3-5/8”

Core dimensions-  10.5” x 9” x 3-5/8”

Volume of water it will hold- 39 oz.

Inlet size- AN16 fitting, or 1” ID

Held still about 5 ft from the outlet

Water temp in- 142 F

Water temp out- 104 F

Pool water temp- 89 F

Moving the rad vigorously for more flow

Water temp out- 101 F

# Test Rad #1

## Single layer (1” borders, ½” dividers- 4 each @ 20.25” long)

## Dimension- 12” X 24”

## Skin thickness- .032”

Channel width- 1-5/8”

Channel height- 1/8”

Volume of water it will hold- 15 oz.

Inlet size- ½” hose barb (smaller ID about 3/8”)

Held still about 5 ft from the outlet

Water temp in- 139 F

Water temp out- 119 F

Pool temp- 89 F

Moving vigorously for more flow

Water temp out- 114 F