Lancair 235/320 103MD Checklist

## PREFLIGHT

INTERIOR

Required Documents	Aboard

Gear switch 		Down

Battery Switch		ON

Electric Fuel Pump	ON, then OFF

(listen to verify operation, confirm pressurization)

Transfer pump		ON, then OFF

(listen to verify operation)

Landing lights, strobe, nav	Check working

Trim tab			Set for takeoff (2/3 forward)

Flaps			Full Down

All Electric Switches	OFF

Battery Switch		OFF

EXTERIOR

Left wing

Left flap			Check hinges & minimal play

Left aileron		Check hinges & minimal play

Left wing top		Check wrinkles & dam

Left wing tip &  light	Check

Left wing fuel vents	Check clear

Left wing tie down	Untie / stow hook

Pitot tube		Check clear

Left fuel tank		Check fuel level if visible

Left landing gear		Check tire, brakes, springs

Empennage

Elevator			Check hinge & minimal play

Elevator nuts		Check

Rudder			Check hinge & attach

Rudder cables		Check

Rudder light		Check

Right wing

Right landing gear	Check tire, brakes, springs

Right wing top		Check
Right wing tie down	Untie / stow hook

Right flap		Check hinges & minimal play

Right aileron		Check hinges & minimal play

Right wing tip & light	Check

Right wing fuel vents	Check clear

AOA upper & lower ports	Check clear

AOA drain		Drain

Right fuel tank		Check fuel level if visible

Right landing gear	Check tire & brake

Cowl/engine

Propeller		Check for damage

Air Inlets			Check for Objects

(oil cooler, air intake, 2 engine cooling)

Top Cowl, Oil Door	Check Oil level, 6-8 qts

Gascolator	Drain

Central fuel vents	Check clear

Exhaust pipes	Clear

Nose gear	Check condition

Nose gear hydraulic lines	Check for wear

Tow Bar	Removed, Stowed

Ground around prop	Clean, free of FOD

### BEFORE STARTING ENGINE

Gear switch		Down

Baggage		Secured

Electrical equipment	Off

Autopilot			Off

Preflight			Complete

Passenger		Briefed

Fire Extinguisher		Shown, demoed

Canopy 			All 4 latched & locked

Seat Belts & Harnesses	Fastened

Circuit Breakers		Set as required

Battery and Alternator	On

Gear			3 Green

AOA			LEDs lit, “AOA Pass”

Fuel boost pump		On/Check Pressure/Off

### STARTING

Toe Brakes		ON

Elevators		Full back

Fuel Selector		FULLEST

Mixture			RICH (IN)

Position Lights		ON

Prop Area		CLEAR

Primer..(IF COLD)	2-3 strokes as required, 0 hot

1@high field elevations

Starter Switch		Turn Key

Throttle			1 full stoke then 1/4"

Caution: If engine fire, continue cranking.  Pull mix full lean. If engine fails to start and fire cont. secure Ign., Bat., and Alt Switches, Fuel Off, Exit

Oil- Pressure		Check Green

If oil fails to exceed 25 psi in 30 secs hot/60 sec cold, shut down.

Alternator		Check Charging

Clock / Timer		Set

### BEFORE TAXI

Mixture			Lean 1”  for ground operation

Radios			On/Set

Circuit breakers		Set as required

Transponder		STBY

Flaps			UP

Oil Temperature		Monitor

ATIS/AI, ALT, DG		Check/SET

Brakes			Test

### BEFORE TAKEOFF

Fuel

Fuel caps	All 3 secure

Boost pump	On

Header tank	Selected

Mixture 	RICH

Primer	In and Locked

Flight Controls		Free & Correct

Primary = A,E,R   Secondary = Trim & Flaps

Flaps			Neutral

Trim Tabs		Set for 2/3 forward

GPS			Set

Runup

Face into wind, nose wheel straight, hold brakes

Throttle		To 1800 RPM

Magnetos 	150 Max drop <50 diff

Carb Heat.	confirm drop

Engine Instrum.	Check Green Arcs

Throttle		Idle

Vacuum Gauge		Check Green

Alternator		Charging, 13.5 – 14.6 V

Flow Check- Fuel, Primer, Mags, Mixture, Carb Heat, Trim, Flaps

Radios			Set

Transponder		Altitude

Strobes			On

Canopy	4 latched

Heading Indicator		Set on runway heading

Lights Camera (transponder) Action (flaps+mix+prop)

## FLIGHT

# TAKEOFF

Flaps			Neutral

Fuel boost pump 		On

Carb Heat		OFF (IN)

Mixture			Full rich unless high altitude

Throttle			Full Open

Tach			2270-2400 RPM

Engine Instruments	CHECK Green

Airspeed indicator	Alive

Rotate @		60-65 kts IAS

No remaining runway	Gear Up

Climb Vx		90 ? kts IAS

Climb Vy		120 ? kts IAS

### CLIMB

Throttle			25” MAP

Engine boost pump	Off

Landing light		Off

Flaps			Reflex

Increase throttle with altitude to maintain MAP

### CRUISE

Throttle			As desired

Trim 			As required

Transfer pumps		To maintain ~10 gal NOSE

If feeding from wings	Switch to NOSE as needed

Leaning			6.6–7.5 gph below 20” MAP

### BEFORE LANDING

G - Fuel Selector		NOSE

U - Gear			Down when <120kts

3 GREEN

(check on downwind, base, final)

M - Mixture Control	Rich (for altitude)

P – Prop			(attached)

S - Seat Belt & Harnesses	Fastened

Flaps			Down full when <90kts

Carb Heat		Pull ON

BALKED LANDING (GO AROUND)

Mixture	Full rich unless high alt.

Throttle 			Full Open (IN)

Carb Heat		COLD (IN)

Flaps			Neutral

Trim	2/3 forward

Cram Clean Climb (Cool) Confess

Speeds

Vso (stall dirty)		46 KIAS

Vs (stall clean)		51 KIAS

Vr (rotate)		65 KIAS

Vfe (max flaps extend)	90 KIAS

Vle (max gear extend)	120 KIAS

Va (maneuvering)	140 KIAS

Vne (redline)		230 KIAS

Vg Best glide		100 KIAS

## POSTFLIGHT

### AFTER LANDING

Flaps			UP while rolling

Carb Heat 		COLD (IN)

Lights			As Required

Transponder		Standby

Mixture			Lean as Required

Fuel pump		OFF

### SHUTDOWN

Radios			OFF

Magneto Ground check	Perform < 1000 rpm

Mixture			FULL Lean->cutoff

Confirm 50rpm rise before cutout

Prop			Horizontal

All Switches		OFF

Control stick		Tie with seatbelt if outside

Tiedowns		Install

Chocks			Install

Tach Time		Note in log

#### EMERGENCY PROCEDURES

Basic Rules		FLY the Plane, Analyze the problem and take Action, Land ASAP

#### ENGINE EMERGENCY SHUTDOWN

Mixture			FULL LEAN (OUT)

Fuel Selector		OFF

Ignition Switch		OFF

#### ENGINE FIRE DURING STARTING

Mixture			FULL LEAN (OUT)

Throttle			OPEN

CONTINUE CRANKING SEVERAL ROTATIONS. Attempt to draw fire inside engine. Accomplish ENGINE EMERG. SHUTDOWN.

#### ENGINE FIRE AFTER STARTING

Engine Emerg. Shutdown

Master Switch		OFF

TAKEOFF ABORT (BEFORE LIFTOFF)

Throttle			CLOSED

Brakes			AS REQUIRED

#### ENGINE FAILURE AFTER TAKEOFF

Glide			100 kts flaps 0

Fuel Selector		NOSE or FULLEST

Electric Fuel pump 	ON

Mixture			RICH

Ignition			ON

Carb Heat		PULL ON

If engine doesn't Start

ENGINE EMERG. SHUTDOWN

Flaps			As Required

Master Switch		OFF

#### PARTIAL POWER FAILURE

Mixture			RICH

Carb Heat		PULL ON

Airspeed			Glide at 100 kt IAS if necessary  to maintain level flight

Fuel Selector		NOSE or WINGS

Electric Fuel Pump	ON

Ignition switch		BOTH

Master Switch		ON

#### COMPLETE POWER FAILURE

Glide			100 kts

Attempt engine restart if warranted

#### ENGINE AIRSTART

Fuel Selector		NOSE or WINGS

Electric Fuel Pump	ON

Mixture			RICH

Ignition switch	BOTH, start if propeller is not turning

Fuel Boost Pump		ON

IF Engine doesn't start, try flooded ENGINE clearing procedure with throttle wide open and mixture full LEAN

If no start, make forced landing

#### ELECTRICAL FIRE

Master Switch 		OFF

Accomplish ENGINE EMERG SHUTDOWN

Make forced Landing.

#### STRUCTURAL DAMAGE

On Takeoff		ABORT

In Flight			Control Airspeed

Climb to safe stall recovery Altitude,

Notify controlling agency,

Make full stop landing with approach speed 5-10 mph higher than normal