=============================================

Bob N. strongly recommends using externally regulated alternators only, or

modifying the internal regulator models so as to allow it to be shut down

if necessary. (Supposedly, a voltage run-away can happen in a few seconds.)

Problem is that the articles I read explaining how to convert the

internally regulated alternators to external regulation didn't work. (Did I

do something wrong?) I think I have worked out a simpler solution. The

objective here is to isolate the field windings from the diode bridge so

that the alternator can then be externally controlled.

This is how I went about converting my alternators. First I removed the

back cover, exposing the diode bridge, regulator assy. and brush holder.

Then removed the regulator and brush holder and threw the regulator assy in

the trash. Next I removed the brush holder and shaped a little brace from

1/8" phenolic that will support one side of the brush holder and isolate it

from the diode bridge. This piece will be about 7/8" x 1/4" with a #8 hole

in each end. Do not make this piece out of metal or it will short out the

diode bridge. Also make up a jumper wire about three inches long with a

ring terminal on each end. One end will attach to the right terminal on the

brush holder and the other will attach to the alternator case (gnd.).

The other terminal on the brush holder will need a spacer under it to keep

everything aligned properly. The left brush will need to have the lead wire

un-soldered from the brush holder. The wire comes through the back of the

brush holder. Unsolder it there and drill out the hole just a bit to allow

a #20 wire with shrink tube to fit snugly through the hole. Solder the #20

wire to the end of the braided wire on the brush.

Now we're ready to reassemble the whole thing. Pass the #20 wire through

the hole drilled in the brush holder and reinsert the spring and brush.

Screw the brush holder back into place, with a spacer washer under the left

terminal. The ground wire goes on the opposite terminal. Feed the wire

soldered to the brush through a hole in the alternator cover, with a

grommet to prevent chaffing. This wire will go to the "F" terminal of the

external voltage regulator. I used a generic Ford unit from a mid-seventies

model (approx. $10/ea from Auto Zone). The S & A terminals of the regulator

tie together and get fed from the alt. switch & 5 amp breaker. The "B"

terminal on the alternator goes to the battery contactor. The other

terminal on the regulator isn't used.

I modified two ND alternators using this method and they both work great.

It takes about 30 minutes to do the modifications. All I need to do now is

add a crowbar over-voltage protector to each regulator and I'm set to go.

Or you can use a B&C voltage regulator with built-in OVP and low-voltage

warning.