SUMMARY OF LANCAIR ACCIDENTS IN NTSB DATABASE

AS OF 8/29/05
Prepared by Lee Metcalfe, N320WH, Lee.Metcalfe@jocoks.com
NOTES: 

“PHASE” is NTSB’s categorization.

“STATISTICAL CLASSIFICATION” is preparer’s categorization for statistical purposes.
“SUMMARY” is preparer’s interpretation of key facts in the NTSB report.

This summary includes all Lancair accidents found in the NTSB database.  There are undoubtedly many minor incidents that did not make it into the database, such as gear-up landings with minor damage and no injuries, in-flight incidents that ended with a safe precautionary or forced landing, etc.  Preparer is personally aware of three gear-up landings and an off-airport emergency landing that are not included in the database.  Preparer personally experienced a prop-strike in a porpoised landing and an in-flight fuel leak in the cockpit that resulted in a safe precautionary landing, neither of which were reportable under NTSB criteria.

	DATE OF ACC.
	LOCATION
	MODEL
	WX
	INJURIES
	PHASE
	STATISTICAL CLASSIFICATION
	SUMMARY

	8/1/89
	Fond du Lac, WI
	235
	VMC
	Fatal (2)
	Approach
	Stall/spin
	Demonstration flight, aircraft stalled and spun during turn from base to final.

	6/23/90
	Davis, CA
	360
	VMC
	Serious
	Take-off
	Fuel
	Pilot observed fuel tank "stopper" back out and depart aircraft, followed by a large quantity of fuel surging out of the tank, engine quit during return to airport, aircraft struck storage building during forced landing.

	2/1/91
	Agua Caliente, CA
	235
	VMC
	Serious
	Climb
	Prop Failure
	Wood propeller blades lost in flight, aircraft crash-landed, forest products lab found that blades fractured and separated due to chemical decomposition of the adhesive to wood bond caused by excessive heat and moisture, name of prop manufacturer is not mentioned in the summary report.

	4/22/91
	Titusville, FL
	360
	VMC
	Fatal (2)
	Descent
	Loss of Power 
	Aircraft experienced a total loss of power and crashed while attempting a forced landing, the alternate air door was found broken off inside the air box and in a position blocking air flow to the throttle body, alternate air door had been made of .032 aluminum (plans called for .040) with a 90* bend at the point of failure, marks inside the air box indicated the door had moved around in the box for some time, air box had no impact damage that would have caused the door to fail.

	5/5/91
	Fall River Mill, CA
	320
	VMC
	None
	Landing
	Crosswind
	Aircraft landed hard and bounced three times before drifting off the left side of the runway, dragging the wing tip and cart-wheeling into a hangar door, pilot reported wind shifting from crosswind to quartering tailwind.

	7/3/92
	Winnemucca, NV
	235
	VMC
	None
	Landing
	Landing
	Pilot lost directional control on landing rollout, veered off runway and struck a raised concrete slab, no mention of wind conditions.

	9/26/92
	Hollister, CA
	235
	VMC
	Fatal (1)
	Approach
	CFIT/O
	Aircraft struck a guy wire 30 feet above ground on low approach, dove into the ground.

	1/14/93
	Madison, WI
	320
	VMC
	None
	Landing
	Landing
	Aircraft collided with snow bank after loss of directional control on landing roll-out, left brake was found to be jammed with frozen snow and ice, aircraft had departed snow-covered runway.

	6/5/93
	Chino, CA
	320
	VMC
	Fatal (3)
	Approach
	Mid-air
	Pilot had been lost, had busted Class C and D airspace and had confused airports, when finally in touch with correct tower was sequenced #3 behind a 172 which was #2 behind a 152, Lancair pilot stated "I have the Cessna, I'm going outside him…", Lancair then struck the 152 in the tail from the left side, Lancair pilot and student and instructor in the 152 died in the subsequent crashes.

	9/22/93
	Rockford. IL
	235
	VMC
	None
	Landing
	Landing
	Landed long on wet grass, braking action nil, attempted abort and struck embankment at end of runway.

	10/10/93
	Bay St. Louis, MS
	320
	VMC
	Serious
	Landing
	Stall/spin
	On second touch and go, touched down left of centerline, bounced, touched down again in left skid, became airborne reaching 60-80 feet in nose-high attitude, stalled and rolled left, collided with trees.

	3/11/94
	Waukesha, WI
	360
	VMC
	None
	Landing
	Landing
	On landing aircraft touched down nose high and bounced, pilot added power but aircraft hard-landed, directional control was lost and aircraft departed runway.

	9/24/94
	Ramona, CA
	235
	VMC
	Fatal (2)
	Takeoff
	Weight & Balance
	Departing for cross-country, aircraft took off 340 lbs. over recommended max gross, aircraft rotated earlier than normal and climbed at unusually high nose-attitude before doing what pilot witness described as a "wing-over" to the left and crashing.

	4/12/95
	Prescott, AZ
	320
	VMC
	None
	Landing
	Weight & Balance
	Aircraft on first test flight after change in configuration for weight & balance, pilot aborted first landing attempt due to inability to stabilize pitch, on second landing attempt during "violent" pitch excursions aircraft stalled and hard-landed, collapsing both mains, pitch trim was found to be set for previous nose-heavy configuration.

	4/18/95
	Hampton, GA
	360
	VMC
	Fatal (1)
	Approach
	Fuel
	Aircraft on downwind after first flight, witnesses reported engine sputtering, nose dropped, engine was powered up and a turn was started toward the airport but aircraft never pulled up, water was found in the carburetor float bowl.

	5/25/95
	Gastonia, NC
	IV-P
	VMC
	Fatal (2)
	Cruise
	In-flight fire
	Aircraft had 16 hours since first flight, during local test flight witnesses reported flames and smoke emanating from the lower cowl, aircraft crashed into trees and burned, cause of in-flight fire could not be determined.

	6/24/95
	Marengo, IL
	320
	VMC
	Fatal (1)
	Take-off
	Pilot Incapacitation
	Aircraft abruptly turned left upon take-off and crashed into trees after turning 240 degrees from runway heading, medical examiner determined pilot had died of heart attack prior to impact.

	8/26/95
	Rio Vista, CA
	320
	VMC
	Serious
	Landing
	Structural Failure
	Asymmetric flap deployment caused loss of roll control on landing, wing contacted runway and aircraft ground-looped, right flap never deployed, flap rod was found bent and rod end broken, Lancair had issued SB to replace flap rods with stronger version but owner had not complied.

	9/3/95
	Georgetown, CA
	200 SE
	VMC
	Fatal (2)
	Take-off
	Tail Wind
	Aircraft started take-off roll with 3-5 knot tailwind, A&P witness said engine was not developing full power, aircraft never lifted off, skid marks started 360' from departure end of runway, aircraft ran off end of runway and down 40 degree slope into trees and burned, throttle and mixture controls were found in full forward position, no mechanical failures or malfunctions found in engine on tear-down, density altitude may have been a factor.

	10/13/95
	Atlanta, GA
	360
	IMC
	Fatal (1)
	Cruise
	Loss of control in IMC
	Non-instrument-rated pilot filed an IFR flight plan, enroute in IMC reported to ATC having "gyro problems" but then said problem was corrected, pilot continued to have difficulty maintaining heading and altitude assignments, witness reported a loud bang followed by aircraft pieces falling out of the clouds, both wings were found 1/4 mile away from main wreckage.

	10/27/95
	Somerset, PA
	IV-P
	VMC
	Serious
	Forced landing
	Engine
	On maiden flight, pilot aborted first landing attempt, on go-around engine "spooled down", aircraft struck power line and trees during forced landing, fuel pump driveshaft had failed.

	11/25/95
	Phoenix, AZ
	360
	VMC
	None
	Landing
	Gear
	During touch and goes, on landing pilot heard thump under aircraft, nose dropped partially and prop struck runway, airplane continued down runway on nose strut until it hit a crack and folded up, pilot reported that the fork to strut attachment had failed.

	12/23/95
	Morris, MN
	320
	VMC
	Fatal (2)
	Maneuvering
	Stall/spin
	Aircraft had 13 hours since first flight, pilot told friend he was going to do stall tests, witnesses saw aircraft stall and spin in, elevator bob weight was found to be installed upside-down, limiting down elevator to 7 degrees versus designed 13 degrees.

	7/20/96
	Cottage Grove, OR
	235
	VMC
	None
	Take-off
	Gear
	Left main folded during take-off in 10-knot quartering right crosswind, aircraft veered off runway and struck parked aircraft, no mechanical failure found in down-lock.

	8/8/96
	Fond du Lac, WI
	320
	VMC
	Fatal (1)
	Cruise
	Prop Failure
	Constant-speed propeller lost in flight due to failure of the crankshaft flange, aircraft CG shifted 3.4 in. aft of aft limit due to loss of prop weight, aircraft entered flat spin into farm field, flange found to have oversized lightening holes and metal softer than spec., crack propagated between lightening holes (this was the Lancair demo plane!)

	10/8/96
	Camarillo, CA
	IV-P
	VMC
	Serious
	Cruise
	Fuel
	Aircraft was flying with a modified fuel selector valve to allow a "both" position, right tank was empty, left had 32 gallons, engine lost power and aircraft crash-landed, fuel valve was bench-tested and found to allow air from the empty tank to leak into fuel flow and lean out the mixture, evidence of engine damage from excessively lean mixture.

	10/12/96
	Kansas City, MO
	235
	VMC
	None
	Cruise
	Fuel
	Departed for local flight with 8-10 gallons of fuel, 15 minutes into flight noticed fuel was almost gone, returning to airport engine quit, crash landed in farm field, tore off all gear legs, found that carburetor float valve was stuck open.

	12/5/96
	Suisun City, CA
	IV-P
	VMC
	Fatal (2)
	Maneuvering
	CFIT/O
	Delivery pilot giving fam ride to new owner, flying southwesterly an hour before sunset, flew into gradually rising terrain in apparent controlled flight.

	7/31/97
	Dillon, CO
	320
	IMC
	Fatal (2)
	Maneuvering
	VFR into IMC
	Non-instrument pilot flying in mountainous area with clouds obscuring mountain tops, flew into box canyon and entered clouds at 12,200 feet, apparent CFIT.

	2/26/98
	Garwood, ID
	IV
	IMC
	Fatal (2)
	Maneuvering
	Loss of control in IMC
	Instrument-rated pilot took off into hard IMC with snow and icing forecast, became disoriented and lost control, no recent instrument time and no instrument time in a Lancair IV.

	4/17/98
	Newport, TN
	235
	MVMC
	Fatal (2)
	Cruise
	VFR into IMC
	Non-instrument pilot flying in mountainous area with clouds obscuring mountain tops, struck ridge, apparent CFIT.

	6/15/98
	Palm Springs, CA
	320
	VMC
	None
	Cruise
	Electrical
	After enroute fuel stop, engine had to be jump-started, after take-off “main” CB tripped and was reset, power continued to drain, all loads shed except transfer pump, flight continued for another 45 minutes until pilot noticed header down to 1/3, initiated precautionary landing, gear extended manually but only right main locked down, made no flap landing and slid off runway, cause of electrical failure not reported.

	7/12/98
	Price, UT
	360
	VMC
	None
	Landing
	Landing
	Pilot stated he allowed descent rate to become excessive, did not add power in time, hard-landed and broke off left main.

	7/15/98
	Atlantic Ocean
	IV-P
	UNK
	Fatal (2)
	UNK
	UNK
	IV-P being ferried from Azores to Newfoundland never arrived, presumed lost at sea.

	9/4/98
	Llano, CA
	235
	IMC
	Fatal (1)
	Cruise
	VFR into IMC
	Non-instrument pilot advised friend in another plane he was turning back due to worse than forecast weather.  Friend saw accident aircraft enter clouds during 180.  Per ATC radar, aircraft wandered around (numerous 180s, 360s and one 540) in IMC for 20 minutes before crashing, witness at crash site reported viz 1/4 mi. in mist and light rain.

	9/21/98
	Reno, NV
	IV
	VMC
	None
	Taxiing
	Taxiing
	Lancair was holding short for take-off, Hawker Sea Fury taxied into his tail!

	5/8/99
	Butte, MT
	IV
	VMC
	None
	Landing
	Landing
	Pilot was practicing short field approaches with high rate of descent, landed hard and collapsed left main, wing struck runway "markers".

	5/11/99
	Tallessee, AL
	320
	VMC
	Fatal (1)
	Take-off
	Stall/spin
	Pilot took off to do a low pass over the runway, during climb out after take-off nose pitched up, aircraft stalled and spun in, engine continued to run at high RPM to impact.

	6/25/99
	Hamilton, OH
	235
	VMC
	Minor
	Landing
	Crosswind
	Landed with relative 100 degree 7-knot crosswind, insufficient rudder authority, pilot rolled left to avoid hangar, wingtip struck ground and aircraft cart-wheeled in.

	8/12/99
	Aspen, CO
	IV-P
	VMC
	None
	Landing
	Tail Wind
	Landed long with a 10-knot tailwind, ran off end of runway, density altitude was over 9,200'.

	12/12/99
	Sterling City, TX
	235
	VMC
	None
	Descent
	Prop
	Wood propeller lost in flight due to overtorquing and fatigue of attach bolts, aircraft struck power lines during forced landing and hard-landed on road.

	4/29/00
	Redding, CA
	235
	VMC
	None
	Take-off
	Crosswind
	Taking off in gusting crosswind, pilot ran out of rudder authority to compensate and lost control, veering off runway.

	6/8/00
	Bountiful, UT
	IV
	VMC
	Minor
	Descent
	Fuel
	Descending from 10,500' to 6,500'  (1,300 agl) pilot pulled power back to 15", when he applied power to level off no response, clipped power lines with the tail on approach to an open field, pilot suspected engine was flooded.

	7/4/00
	Hot Springs, SD
	235
	VMC
	None
	Take-off
	Tail Wind
	Wind shifted between weather briefing and take-off, pilot did not check windsock before taxiing, attempted take-off with 8 knot tailwind, ran off end of runway, density altitude was 4613'.

	8/10/00
	Chiloquin, OR
	235
	VMC
	None
	Forced landing
	Engine
	During cruise engine began running rough, pilot made precautionary landing and had timing adjusted, took off for test flight and engine began running rough again, unable to make airport, pilot landed on the back of a semi on the highway, engine torn down and found to have fractured crankshaft between #3 and #4 crankpins, O-290-G engine was originally from a ground power unit.

	8/15/00
	Artesia, NM
	IV
	VMC
	None
	Take-off
	Take-off
	Upon lift-off aircraft entered a steep bank to the right at 20' agl, right wing contacted the ground, aircraft impacted ground and gear collapsed.

	8/20/00
	Moraine, OH
	IV-P
	VMC
	Serious
	Approach
	Landing
	Aircraft struck a 15' levee 50' short of the threshold of the intended runway, pilot was unaware of levee or 225' displaced threshold, said he could not see runway markings due to pitch attitude of aircraft, pilot had 127 hrs. in the accident aircraft.

	8/31/00
	Bend, OR
	320
	VMC
	None
	Landing
	Crosswind
	Landing in gusting winds, aircraft ballooned, pilot “added some power" but aircraft settled back down off the side of the runway, prop struck the ground and gear collapsed.

	9/27/00
	Kernville, CA
	360
	VMC
	None
	Landing
	Landing
	Aircraft dropped to the runway from 5 feet, bounced and veered off runway striking some large rocks and sheering left main.

	3/4/01
	Marana, AZ
	ES
	VMC
	Minor
	Forced landing
	Engine
	During cruise pilot noticed smoke coming out of cowling, initiated precautionary landing, prop would not respond to power increases, aircraft nosed over in off-airport landing.  Aircraft was equipped with a marine conversion V-8 with belt drive, belt had melted due to overheated bearing in belt-drive unit.

	5/6/01
	Valpariso, IN
	320
	VMC
	Fatal (1)
	Take-off
	No/Low Lancair Time
	Aircraft accelerated slower than normal on take-off, was pitching up and down as it slowly climbed to about 200', rolled left and nose-dived to the ground, was pilot's third flight since completing plane, no other Lancair time.

	5/28/01
	Bennett, CO
	IV-P
	VMC
	Minor
	Landing
	Power Loss
	Lost oil pressure in cruise, forced landing into field, gear buckled and right wing sheered, starter adapter drive shaft had fractured allowing oil to be pumped overboard.

	6/16/01
	Flagstaff, AZ
	IV-P
	Night VMC
	Fatal (1)
	Landing
	Fuel
	During approach after a 4-hour cross-country pilot advised tower he would not make the airport and crashed 2 miles out.  Previous owner of the aircraft said there had been problems filling tanks to design capacity 80 gallons.  It was determined that no vent had been installed in the right wing tank.

	7/6/01
	La Junta, CO
	360
	VMC
	None
	Landing
	Gear
	Gear was lowered and indicated "green" but retracted on landing, "up" relay was found to be electrically “hot,” a mouse nest was found in the center console among the wiring to the gear and flap motors.

	7/11/01
	Kissimmee, FL
	IV
	VMC
	Fatal (1)
	Take-off
	Maneuvering
	Took off for pattern work, turning to downwind aircraft slowed rapidly, “made a couple of abrupt moves side to side,” veered away from the airport and nose-dived into the ground.

	7/23/01
	Fryeburg, ME
	IV-P
	VMC
	None
	Forced landing
	Power Loss
	Engine lost power during cruise, pilot did not get gear down in time during stretched glide to airport, aircraft bellied in, found paper towel in intake manifold and pieces of paper towel in turbocharger, engine inspection and cleaning had occurred just prior to accident flight.

	7/27/01
	Charleston, N.Y.
	320
	VMC
	Serious
	Maneuvering
	Low Pass
	Pilot made several low passes over friend's private airfield, during steep turn back to airport descended into trees.

	7/31/01
	St. Mary's, GA
	360
	VMC
	Fatal (2)
	Maneuvering
	Loss of control in IMC
	Student and CFI on IFR flight plan at 6000' talking to ATC, dodging heavier build-ups, entered a squall line off-shore, radar showed rapidly descending turn followed by crash into ocean.

	8/17/01
	Placerville, CA
	350
	VMC
	Fatal (2)
	Maneuvering
	Low Pass
	Pilot made a low pass over his home then crashed in neighbor's yard for unknown reasons.  Pilot was a 15,000 hr. ATP.

	12/5/01
	Tecumseh, MI
	235
	VMC
	None
	Take-off
	Crosswind
	Taking off in gusting crosswind, wing contacted runway and pilot lost control.

	5/16/02
	Hattiesburg, MS
	Super ES
	VMC
	Fatal (2)
	Take-off
	Power Loss
	During a touch-and-go, witnesses heard a popping sound, saw aircraft in steep right turn, following 3 more rapid pops aircraft nosed down into wooded area and exploded.  Pilot had traces of marijuana in blood and urine.

	7/13/02
	Sedona, AZ
	320
	VMC
	None
	Landing
	Crosswind
	During crosswind landing left wing contacted ground, followed by prop and nose.

	8/15/02
	Iola, KS
	320
	IMC
	Minor
	Forced landing
	Power Loss
	On instrument approach, lost power, restart unsuccessful, landed in farm field, hit berm and nosed over.

	10/15/02
	Coeur d'Alene, ID
	IV
	VMC
	Serious (1)

Fatal (2)
	Landing
	Landing
	Very experienced Lancair pilot unfamiliar with private 2200' x 40' runway with trees on either side, flew unstable final, touched down with right main off pavement and veered into trees.

	11/5/02
	Bozeman, MT
	IV-P
	VMC
	Fatal (1)
	Approach
	Power Loss
	On 5-mile visual approach after 3-hour-plus cross-country, pilot reported losing power, witness reported a steep right turn followed by steep dive into a field just short of a housing development.  Fuel valve was found to be between detents.

	11/21/02
	Belverde, TX
	IV-P
	VMC
	None
	Landing
	No/Low Lancair Time
	During third attempt to land aircraft departed runway under heavy braking 2,300’ down a 3,000' runway, struck runway lights, ditch & fence.

	12/20/02
	Geelong, AUS
	IV
	VMC
	Fatal (2)
	Maneuvering
	Stall/spin
	Fifth flight since completion, performing stall sequences, failed to recover from stall/spin.

	12/28/02
	Payson, AZ
	235
	VMC
	Fatal (1)
	Aborted landing
	No/Low Lancair Time
	Pilot had just bought plane, no Lancair training, first flight, first and second landing attempts aborted, stall/spin on third abort.

	12/30/02
	Corona, CA
	320
	VMC
	None
	Cruise
	Engine
	Lost oil pressure in cruise, forced landing on freeway, oil line had come loose, oil pumped overboard.

	4/26/03
	Wamego, KS
	IV
	VMC
	None
	Final
	Landing
	S-turning on short final for spacing behind slower aircraft, inadvertently contacted runway while turning.

	5/16/03
	Allendale, SC
	IV
	IMC
	Fatal (4)
	Cruise
	Loss of control in IMC
	IFR cross-country, maneuvering to avoid thunderstorms, broke up in flight.

	7/26/03
	Page, AZ
	235
	VMC
	None
	Landing
	Landing
	On landing aircraft porpoised and landed hard on nose gear, which collapsed.  Aircraft skidded on nose, fuel line broke and fire started.

	7/27/03
	North Las Vegas, NV
	IV-P
	VMC
	Minor
	Take-off
	Take-off
	After being warned of possible wake turbulence from preceding aircraft, pilot attempted take-off and lost control, landing hard and collapsing gear.


	7/30/03
	Fort Payne, AL
	235
	VMC
	Minor
	Landing
	Engine
	After local flight of unreported duration, engine quit on final, crashed into trees.

	8/1/03
	Oshkosh, WI
	360
	VMC
	None
	Landing
	Landing
	During EAA convention, RV had just landed on right side of runway, Lancair was landing on left side of runway per ATC instructions, RV turned left in front of Lancair and the two collided.

	8/7/03
	Vancouver, WA
	360
	VMC
	None
	Forced landing
	Engine
	Climbing out after take-off pilot noticed a puff of smoke from cowling, elected to return to airport, reduced power to descend and did 180, then added power to level off for traffic pattern, engine didn’t respond, landed on nearby road and struck rear of garbage truck, aircraft burned after pilot exited.

	9/2/03
	Manassas, VA
	IV-P
	VMC
	None
	Take-off
	Take-off
	Pilot distracted by copying complex IFR clearance during pre-take-off check, forgot to extend flaps to take-off position, during rotation aircraft settled back on runway, drifted off the right side of the runway and struck a sign, came to rest upright in grass.

	9/19/03
	Kimberling City, MO
	Legacy
	VMC
	None
	Take-off
	Take-off
	Pilot had placed temporary paper cover on pitot tube with no flag.  Forgot to remove prior to flight.  During attempted take-off from 2,600 ft. runway with trees at departure end, noticed abnormally low airspeed, continued to accelerate and rotate, got aural AOA warning, decided to abort, could not stop before hitting trees.

	10/20/03
	Knoxville, TN
	360
	VMC
	Serious (1) 
Fatal (1)
	Forced Landing
	Loss of Power
	Immediately after take-off engine began to run rough, as pilot maneuvered to return to airport, engine quit, plane crashed into rocky embankment a few yards short of runway.

	11/10/03
	Memphis, TN
	235
	VMC
	None
	Forced Landing
	Loss of Power
	Immediately after take-off engine lost power, pilot landed on highway adjacent to airport.

	2/17/04
	Mesa, AZ
	IV-P-T
	VMC
	Fatal (2)
	Maneuvering
	Stall/Spin
	Turbine IV-P on third test flight.  Pilot witness saw aircraft in a flat spin before impact.

	4/7/04
	Novalato, Mexico
	IV-P
	VMC
	Fatal (3)
	Maneuvering
	Stall/Spin
	Mexican owner purchased American-built IV-P, receiving qualification instruction from an American CFI using an interpreter, witnesses reported stall/spin.

	4/23/04
	Memphis, TN
	Legacy
	IMC
	Fatal
	Cruise
	Loss of control in IMC*
	Pilot was being vectored by ATC to avoid thunderstorms.  ATC issued a recommended heading through a band of cells which pilot followed, ended up flying into a Level 5 cell and losing control.  Aircraft was seen in a descending right turn before disappearing from radar.  Wreckage indicated aircraft was intact when it struck the ground.  Editorial note:  a person familiar with this airplane reported on the LML that the only attitude indicator this plane had was the Dynon D-10, for which, according to Dynon, “maximum rates of change in attitude are limited to 150 degrees/second in roll, pitch, and yaw. Aircraft maneuvers outside this envelope may result in an inaccurate display and a warning message is shown. The instrument will return to specified accuracy within 10 seconds after the aircraft returns to coordinated flight with rates of change in attitude of no more than 4 degrees/second.”

	5/18/04
	Grand Canyon, AZ
	IV-P
	VMC
	Fatal (4)
	Maneuvering
	Stall/spin
	Aircraft fell out of sky during sight-seeing flight over Grand Canyon.  Witnesses reported aircraft in spin prior to impact.  Pilot and three adult pax.

	5/31/04
	Vermontville, MI
	IV-P
	IMC
	Fatal (3)
	Cruise
	Loss of control in IMC
	IFR cross-country, pilot reported unable to maintain altitude, witness reported seeing plane in flat spin just prior to impact.  Radar review showed aircraft entering an area of Level 6 precip followed by rapid descent.  ATC never advised pilot of the Level 5 & 6 cells.

	6/13/04
	Hays, KS
	235
	VMC
	None
	Landing
	Crosswind
	Departed runway on landing roll-out due to loss of control in 30 degree crosswind, 23 kts. gusting to 28 kts. 

	7/12/04
	Pacoima, CA
	360
	VMC
	None
	Landing
	Tail Wind
	Pilot acknowledged a 10-knot tailwind warning, touched down 1,800’ down a 4,125’ runway, locked brakes on the last 400’ of runway, ran off end into fence.  DUH!

	8/1/04
	Madison, WI
	Legacy
	VMC
	Fatal (1)
	Forced Landing
	Engine failure
	Supercharged Legacy leaving Oshkosh, pilot previously commented to others he had been seeing high engine temps, reported engine problems to ATC 10 minutes into flight at 12,000’, diverted to Dane Co. Regional Airport 10 miles away, failed to make airport, crashed and burned after hitting light pole during engine-out approach to a grassy strip adjacent to a street.  Engine teardown revealed one piston burned through and other heat damage not related to post-crash fire.  Pilot overflew another airport enroute to Dane Co.

	8/27/04
	Houston, TX
	320
	VMC
	None
	Take-off
	Take-off
	Pilot reported he over-rotated on take-off, aircraft settled back onto runway drifting left due to right quartering crosswind, struck edge lights, taxiway sign and culvert, nose gear collapsed.

	9/3/04
	Chewelah, WA
	320
	VMC
	Minor (2)
	Maneuvering
	Engine failure
	Pilot reported this was first flight after engine rebuild, orbited over airport at 75% power for 30 minutes, maneuvering for landing, lowered gear and smelled strong odor of fuel, engine sputtered and quit, couldn’t make runway, crash-landed in open field.

	9/4/04
	Port Townsend, WA
	360
	VMC
	None
	Landing
	Landing Gear
	On sixth touch-and-go, pilot forgot to lower gear.

	9/10/04
	Orange, TX
	360
	VMC
	Fatal (1)
	Maneuvering
	Stall/spin
	CFI witness reported seeing aircraft doing a low pass over airport with gear down (unknown reason), followed by pull up, and then what appeared to be a power-on stall, recovery, then high speed impact
with terrain.

	9/12/04
	Cambridge, OH
	Legacy
	VMC
	None
	Maneuvering
	Structural Failure
	Upper skin of left stub wing departed aircraft while in a left turn, aircraft landed safely.  Examination showed poor bonding between skin and wing structure.

	9/26/04
	Sylvester, GA
	IV-P-T
	VMC
	None
	Landing
	Loss of Power
	Turbine IV-P entering downwind for landing, pilot added throttle to arrest descent, engine did not respond, plane crash-landed short of runway and skidded into parked aircraft on ramp.

	10/20/04
	Las Cruces, NM
	360
	VMC
	None
	Take-off
	Loss of Control
	Pilot reported that as he rotated for take-off, the aircraft “turned right” and departed the runway.  The left main and nose gear were sheared off and the left wing “bent” aft.

	1/5/05
	Sacaton, AZ
	235
	VMC
	Fatal (2)
	Maneuvering
	Stall/spin
	Witness reported watching the plane doing maneuvers when it “zoomed up” in a nose-high attitude, a wing dropped and the plane entered a spin from which it did not recover prior to impact.

	1/12/05
	El Cajon, CA
	235
	VMC
	Serious (1) 

Fatal (1)
	Landing
	CFIT/O
	Aircraft apparently on short final for landing struck the airport perimeter fence 500’ short of the runway and impacted the ground.  A passenger survived.

	4/8/05
	Bend, OR
	Legacy
	VMC
	None
	Landing
	Landing Gear
	Pilot forgot to lower gear for landing.

	5/1/05
	Nacodoches, TX
	Legacy
	VMC
	None
	Maneuvering
	Loss of Power
	While maneuvering at 700-800’ agl, engine lost power.  Pilot “traded airspeed for altitude” and attempted to regain power, but was unable.  Pilot attempted a landing in an open field.  As the plane touched down the main gear sheered off and the plane skidded to a stop.  A post-crash fire consumed the plane after the pilot and passenger exited the plane.  Plane had accumulated 30 hours since completion.

	8/9/05
	Sundre, Alberta, Canada
	IV-P-T
	IMC
	Fatal (2)
	Cruise
	Loss of Control in IMC
	IFR cross-country, flew into an area of strong storms, broke up in flight.

	8/15/05
	Madras, OR
	Legacy FG
	VMC
	Fatal (2)
	Maneuvering
	Stall/spin
	Witnesses reported hearing a sputtering engine and seeing the plane in a spin prior to impact.


