TK-5 Specifications and Parts List
· Weight: 1.15 lbs. (523 grams) 
· Length: user adjustable from 5.790" to 6.0". 
· Pressure: 300psi 20.7bar (operating pressure: 250psi – 350psi)



[image: image1.png]6010" - 5750





	# 
	Part #
	Drawing
	Supplier

	 
	 
	 
	 

	1
	21144
	Shaft
	Leading Edge

	2
	21153
	Damping Piston
	Leading Edge

	3
	21155
	Body
	Leading Edge

	4
	21156
	Bearing 
	Leading Edge

	5
	21143
	Shaft Eyelet
	Leading Edge

	6
	21162
	Internal Floating Piston
	Leading Edge

	7
	20109
	Piston Nut
	Leading Edge

	8
	21162
	Eccentric Bushing (2)
	Leading Edge

	9
	21165
	Schrader Valve Body
	Schrader

	10
	2525
	Schrader Valve Cap
	Schrader

	11
	05DU04
	DU Bushing Eyelet (2)
	Garlock

	12
	05DU08
	DU Bushing Eccentric (2)
	Garlock

	13
	N/A
	10-32 x 1.0 SS 8.8 SHCS (2)
	McMaster Carr

	14
	AN365-1032A
	10-32 Nylock Nut (2)
	Aircraft Spruce

	15
	2-137
	Bearing Outer O-Ring (N70)
	Parker

	16
	Q-226
	Bearing Inner Q-Ring (N70)
	Parker

	17
	8600-0200
	Bearing Wiper (N70)
	Parker

	18
	Q-222
	IFP Q-Ring (2) (N70)
	Parker

	19
	2-031
	Eyelet O-Ring (N70)
	Parker

	20
	2-010
	Schrader Valve O-Ring (N70)
	Parker

	21
	6035
	Shrader Valve Core
	Schrader



TimCo Engineering, LLC  374 SE Soft Tail Loop  Bend, OR   97702  (541) 317-1415

timothy.ong@leadingedgeaircraft.com
www.leadingedgeaircraft.com

