SUMMARY OF LANCAIR ACCIDENTS IN NTSB DATABASE

AS OF 10/24/2011

Prepared by Lee Metcalfe, former LNC2 owner/pilot, Lee.Metcalfe@jocogov.org

NOTES:

“PHASE” is NTSB’s categorization of phase of flight.

“STATISTICAL CLASSIFICATION” is preparer’s categorization for statistical purposes.

“SUMMARY” is preparer’s interpretation of key facts in the NTSB report.

This summary includes all Lancair accidents found in the NTSB database.  There are undoubtedly many minor incidents that did not make it into the database, such as gear-up landings with minor damage and no injuries, in-flight incidents that ended with a safe precautionary or forced landing, etc.  Preparer is personally aware of three gear-up landings and an off-airport emergency landing that are not included in the database.  Preparer personally experienced a prop-strike in a porpoised landing and an in-flight fuel leak in the cockpit that resulted in a safe precautionary landing, neither of which were reportable under NTSB criteria.

| DATE OF ACC. | LOCATION | MODEL | WX | INJURIES | PHASE | STATISTICAL CLASSIFICATION | SUMMARY |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8/1/89 | Fond du Lac, WI | 235 | VMC | Fatal (2) | Approach | Stall/spin | Demonstration flight, aircraft stalled and spun during turn from base to final for landing. |
| 6/23/90 | Davis, CA | 360 | VMC | Serious | Take-off | Fuel | Pilot observed fuel tank "stopper" back out and depart aircraft, followed by a large quantity of fuel surging out of the tank, engine quit during return to airport, aircraft struck storage building during forced landing. |
| 2/1/91 | Agua Caliente, CA | 235 | VMC | Serious | Forced landing | Prop Failure | Wood propeller blades lost in flight, aircraft crash-landed, forest products lab found that blades fractured and separated due to chemical decomposition of the adhesive to wood bond caused by excessive heat and moisture, name of prop manufacturer is not mentioned in the summary report. |
| 4/22/91 | Titusville, FL | 360 | VMC | Fatal (2) | Forced landing | Loss of Power | Aircraft experienced a total loss of power and crashed while attempting a forced landing, the alternate air door was found broken off inside the air box and in a position blocking air flow to the throttle body, alternate air door had been made of .032 aluminum (plans called for .040) with a 90* bend at the point of failure, marks inside the air box indicated the door had moved around in the box for some time, air box had no impact damage that would have caused the door to fail. |
| 5/5/91 | Fall River Mill, CA | 320 | VMC | None | Landing | Crosswind | Aircraft landed hard and bounced three times before drifting off the left side of the runway, dragging the wing tip and cart-wheeling into a hangar door, pilot reported wind shifting from crosswind to quartering tailwind. |
| 7/3/92 | Winnemucca, NV | 235 | VMC | None | Landing | Landing | Pilot lost directional control on landing rollout, veered off runway and struck a raised concrete slab, no mention of wind conditions. |
| 9/26/92 | Hollister, CA | 235 | VMC | Fatal (1) | Approach | CFIT/O | Aircraft struck a guy wire 30 feet above ground on low approach, dove into the ground. |
| 1/14/93 | Madison, WI | 320 | VMC | None | Landing | Landing | Aircraft collided with snow bank after loss of directional control on landing roll-out, left brake was found to be jammed with frozen snow and ice, aircraft had departed snow-covered runway. |
| 6/5/93 | Chino, CA | 320 | VMC | Fatal (3) | Approach | Mid-air | Pilot had been lost, had busted Class C and D airspace and had confused airports, when finally in touch with correct tower was sequenced #3 behind a 172 which was #2 behind a 152, Lancair pilot stated "I have the Cessna, I'm going outside him…", Lancair then struck the 152 in the tail from the left side, Lancair pilot and student and instructor in the 152 died in the subsequent crashes. |
| 9/22/93 | Rockford. IL | 235 | VMC | None | Landing | Landing | Landed long on wet grass, braking action nil, attempted abort and struck embankment at end of runway. |
| 10/10/93 | Bay St. Louis, MS | 320 | VMC | Serious | Landing | Stall/spin | On second touch and go, touched down left of centerline, bounced, touched down again in left skid, became airborne reaching 60-80 feet in nose-high attitude, stalled and rolled left, collided with trees. |
| 3/11/94 | Waukesha, WI | 360 | VMC | None | Landing | Landing | On landing aircraft touched down nose high and bounced, pilot added power but aircraft hard-landed, directional control was lost and aircraft departed runway. |
| 9/24/94 | Ramona, CA | 235 | VMC | Fatal (2) | Takeoff | Weight & Balance | Departing for cross-country, aircraft took off 340 lbs. over recommended max gross, aircraft rotated earlier than normal and climbed at unusually high nose-attitude before doing what pilot witness described as a "wing-over" to the left and crashing. |
| 4/12/95 | Prescott, AZ | 320 | VMC | None | Landing | Weight & Balance | Aircraft on first test flight after change in configuration for weight & balance, pilot aborted first landing attempt due to inability to stabilize pitch, on second landing attempt during "violent" pitch excursions aircraft stalled and hard-landed, collapsing both mains, pitch trim was found to be set for previous nose-heavy configuration. |
| 4/18/95 | Hampton, GA | 360 | VMC | Fatal (1) | Approach | Fuel | Aircraft on downwind after first flight, witnesses reported engine sputtering, nose dropped, engine was powered up and a turn was started toward the airport but aircraft never pulled up, water was found in the carburetor float bowl. |
| 5/25/95 | Gastonia, NC | IV-P | VMC | Fatal (2) | Cruise | In-flight fire | Aircraft had 16 hours since first flight, during local test flight witnesses reported flames and smoke emanating from the lower cowl, aircraft crashed into trees and burned, cause of in-flight fire could not be determined. |
| 6/24/95 | Marengo, IL | 320 | VMC | Fatal (1) | Take-off | Pilot Incapacitation | Aircraft abruptly turned left upon take-off and crashed into trees after turning 240 degrees from runway heading, medical examiner determined pilot had died of heart attack prior to impact. |
| 8/26/95 | Rio Vista, CA | 320 | VMC | Serious | Landing | Structural Failure | Asymmetric flap deployment caused loss of roll control on landing, wing contacted runway and aircraft ground-looped, right flap failed to deploy, flap actuating rod was found bent and rod end broken, Lancair had issued SB to replace flap rods with stronger version but owner had not complied. |
| 9/3/95 | Georgetown, CA | 200 SE | VMC | Fatal (2) | Take-off | Tail Wind | Aircraft started take-off roll with 3-5 knot tailwind, A&P witness said engine was not developing full power, aircraft never lifted off, skid marks started 360' from departure end of runway, aircraft ran off end of runway and down 40 degree slope into trees and burned, throttle and mixture controls were found in full forward position, no mechanical failures or malfunctions found in engine on tear-down, density altitude may have also been a factor. |
| 10/13/95 | Atlanta, GA | 360 | IMC | Fatal (1) | Cruise | Loss of control in IMC | Non-instrument-rated pilot filed an IFR flight plan, enroute in IMC reported to ATC having "gyro problems" but then said problem was corrected, pilot continued to have difficulty maintaining heading and altitude assignments, witness reported a loud bang followed by aircraft pieces falling out of the clouds, both wings were found 1/4 mile away from main wreckage. |
| 10/27/95 | Somerset, PA | IV-P | VMC | Serious | Forced landing | Engine | On maiden flight, pilot aborted first landing attempt, on go-around engine "spooled down", aircraft struck power line and trees during forced landing, fuel pump driveshaft had failed. |
| 11/25/95 | Phoenix, AZ | 360 | VMC | None | Landing | Gear | During touch and goes, on landing pilot heard thump under aircraft, nose dropped partially and prop struck runway, airplane continued down runway on nose strut until it hit a crack and folded up, pilot reported that the fork to strut attachment had failed. |
| 12/23/95 | Morris, MN | 320 | VMC | Fatal (2) | Maneuvering | Stall/spin | Aircraft had 13 hours since first flight, pilot told friend he was going to do stall tests, witnesses saw aircraft stall and spin in, elevator bob weight was found to be installed upside-down, limiting down elevator to 7 degrees versus designed 13 degrees. |
| 7/20/96 | Cottage Grove, OR | 235 | VMC | None | Take-off | Gear | Left main folded during take-off in 10-knot quartering right crosswind, aircraft veered off runway and struck parked aircraft, no mechanical failure found in down-lock. |
| 8/8/96 | Fond du Lac, WI | 320 | VMC | Fatal (1) | Cruise | Prop Failure | Constant-speed propeller lost in flight due to failure of the crankshaft flange, aircraft CG shifted 3.4 in. aft of aft limit due to loss of prop weight, aircraft entered flat spin into farm field, flange found to have oversized lightening holes and metal softer than spec., crack propagated between lightening holes (this was the Lancair demo plane!) |
| 10/8/96 | Camarillo, CA | IV-P | VMC | Serious | Cruise | Fuel | Aircraft was flying with a modified fuel selector valve to allow a "both" position, right tank was empty, left had 32 gallons, engine lost power and aircraft crash-landed, fuel valve was bench-tested and found to allow air from the empty tank to leak into fuel flow and lean out the mixture, evidence of engine damage from excessively lean mixture. |
| 10/12/96 | Kansas City, MO | 235 | VMC | None | Cruise | Fuel | Departed for local flight with 8-10 gallons of fuel, 15 minutes into flight noticed fuel was almost gone, returning to airport engine quit, crash landed in farm field, tore off all gear legs, found that carburetor float valve was stuck open. |
| 12/5/96 | Suisun City, CA | IV-P | VMC | Fatal (2) | Maneuvering | CFIT/O | Delivery pilot giving fam ride to new owner, flying southwesterly an hour before sunset, flew into gradually rising terrain in apparent controlled flight. |
| 7/31/97 | Dillon, CO | 320 | IMC | Fatal (2) | Maneuvering | VFR into IMC | Non-instrument pilot flying in mountainous area with clouds obscuring mountain tops, flew into box canyon and entered clouds at 12,200 feet, apparent CFIT. |
| 2/26/98 | Garwood, ID | IV | IMC | Fatal (2) | Maneuvering | Loss of control in IMC | Instrument-rated pilot took off into hard IMC with snow and icing forecast, became disoriented and lost control, no recent instrument time and no instrument time in a Lancair IV. |
| 4/17/98 | Newport, TN | 235 | IMC | Fatal (2) | Cruise | VFR into IMC | Non-instrument pilot flying in mountainous area with clouds obscuring mountain tops, struck ridge, apparent CFIT. |
| 6/15/98 | Palm Springs, CA | 320 | VMC | None | Cruise | Electrical | After enroute fuel stop, engine had to be jump-started, after take-off “main” CB tripped and was reset, power continued to drain, all loads shed except transfer pump, flight continued for another 45 minutes until pilot noticed header down to 1/3, initiated precautionary landing, gear extended manually but only right main locked down, made no flap landing and slid off runway, cause of electrical failure not reported. |
| 7/12/98 | Price, UT | 360 | VMC | None | Landing | Landing | Pilot stated he allowed descent rate to become excessive, did not add power in time, hard-landed and broke off left main. |
| 7/15/98 | Atlantic Ocean | IV-P | UNK | Fatal (2) | UNK | UNK | IV-P being ferried from Azores to Newfoundland never arrived, presumed lost at sea. |
| 9/4/98 | Llano, CA | 235 | IMC | Fatal (1) | Cruise | VFR into IMC | Non-instrument pilot advised friend in another plane he was turning back due to worse than forecast weather. Friend saw accident aircraft enter clouds during 180. Per ATC radar, aircraft wandered around (numerous 180s, 360s and one 540) in IMC for 20 minutes before crashing, witness at crash site reported viz 1/4 mi. in mist and light rain. |
| 9/21/98 | Reno, NV | IV | VMC | None | Taxiing | Taxiing | Lancair was holding short for take-off, Hawker Sea Fury taxied into his tail! |
| 5/8/99 | Butte, MT | IV | VMC | None | Landing | Landing | Pilot was practicing short field approaches with high rate of descent, landed hard and collapsed left main, wing struck runway "markers". |
| 5/11/99 | Tallassee, AL | 320 | VMC | Fatal (1) | Take-off | Stall/spin | Pilot took off to do a low pass over the runway, during climb out after take-off nose pitched up, aircraft stalled and spun in, engine continued to run at high RPM to impact. |
| 6/25/99 | Hamilton, OH | 235 | VMC | Minor | Landing | Crosswind | Landing with relative 100 degree 7-knot crosswind, insufficient rudder authority, pilot rolled left to avoid hangar, wingtip struck ground and aircraft cart-wheeled in. |
| 8/12/99 | Aspen, CO | IV-P | VMC | None | Landing | Tail Wind | Landed long with a 10-knot tailwind, ran off end of runway, density altitude was over 9,200'. |
| 12/12/99 | Sterling City, TX | 235 | VMC | None | Forced landing | Prop failure | Wood propeller lost in flight due to over-torquing and fatigue of attach bolts, aircraft struck power lines during forced landing and hard-landed on road. |
| 4/29/00 | Redding, CA | 235 | VMC | None | Take-off | Crosswind | Taking off in gusting crosswind, pilot ran out of rudder authority to compensate and lost control, veering off runway. |
| 6/8/00 | Bountiful, UT | IV | VMC | Minor | Descent | Fuel | Descending from 10,500' to 6,500' (1,300 agl) pilot pulled power back to 15", when he applied power to level off no response, clipped power lines with the tail on approach to an open field, pilot suspected engine was flooded. |
| 7/4/00 | Hot Springs, SD | 235 | VMC | None | Take-off | Tail Wind | Wind shifted between weather briefing and take-off, pilot did not check windsock before taxiing, attempted take-off with 8 knot tailwind, ran off end of runway, density altitude was 4613'. |
| 8/10/00 | Chiloquin, OR | 235 | VMC | None | Forced landing | Engine | During cruise engine began running rough, pilot made precautionary landing and had timing adjusted, took off for test flight and engine began running rough again, unable to make airport, pilot landed on the back of a semi on the highway, engine torn down and found to have fractured crankshaft between #3 and #4 crankpins, O-290-G engine was originally from a ground power unit and not certificated for flight. |
| 8/15/00 | Artesia, NM | IV | VMC | None | Take-off | Take-off | Upon lift-off aircraft entered a steep bank to the right at 20' agl, right wing contacted the ground, aircraft impacted ground and gear collapsed. |
| 8/20/00 | Moraine, OH | IV-P | VMC | Serious | Approach | Landing | Aircraft struck a 15' levee 50' short of the threshold of the intended runway, pilot was unaware of levee or 225' displaced threshold, said he could not see runway markings due to pitch attitude of aircraft, pilot had 127 hrs. in the accident aircraft. |
| 8/31/00 | Bend, OR | 320 | VMC | None | Landing | Crosswind | Landing in gusting winds, aircraft ballooned, pilot “added some power" but aircraft settled back down off the side of the runway, prop struck the ground and gear collapsed. |
| 9/27/00 | Kernville, CA | 360 | VMC | None | Landing | Landing | Aircraft dropped to the runway from 5 feet, bounced and veered off runway striking some large rocks and sheering left main. |
| 3/4/01 | Marana, AZ | ES | VMC | Minor | Forced landing | Engine | During cruise pilot noticed smoke coming out of cowling, initiated precautionary landing, prop would not respond to power increases, aircraft nosed over in off-airport landing. Aircraft was equipped with a marine conversion V-8 with belt drive, belt had melted due to overheated bearing in belt-drive unit. |
| 5/6/01 | Valpariso, IN | 320 | VMC | Fatal (1) | Take-off | No/Low Lancair Time | Aircraft accelerated slower than normal on take-off, was pitching up and down as it slowly climbed to about 200', rolled left and nose-dived to the ground, was pilot's third flight since completing plane, no other Lancair time. |
| 5/28/01 | Bennett, CO | IV-P | VMC | Minor | Forced landing | Power Loss | Lost oil pressure in cruise, forced landing into field, gear buckled and right wing sheered, starter adapter drive shaft had fractured allowing oil to be pumped overboard. |
| 6/16/01 | Flagstaff, AZ | IV-P | Night VMC | Fatal (1) | Landing | Fuel | During approach after a 4-hour cross-country pilot advised tower he would not make the airport and crashed 2 miles out. Previous owner of the aircraft said there had been problems filling tanks to design capacity 80 gallons. It was determined that no vent had been installed in the right wing tank. |
| 7/6/01 | La Junta, CO | 360 | VMC | None | Landing | Gear | Gear was lowered and indicated "green" but retracted on landing, "up" relay was found to be electrically “hot,” a mouse nest was found in the center console among the wiring to the gear and flap motors. |
| 7/11/01 | Kissimmee, FL | IV | VMC | Fatal (1) | Take-off | Maneuvering | Took off for pattern work, turning to downwind aircraft slowed rapidly, “made a couple of abrupt moves side to side,” veered away from the airport and nose-dived into the ground. |
| 7/23/01 | Fryeburg, ME | IV-P | VMC | None | Forced landing | Power Loss | Engine lost power during cruise, pilot did not get gear down in time during stretched glide to airport, aircraft bellied in, found paper towel in intake manifold and pieces of paper towel in turbocharger, engine inspection and cleaning had occurred just prior to accident flight. |
| 7/27/01 | Charleston, N.Y. | 320 | VMC | Serious | Maneuvering | Low Pass | Pilot made several low passes over friend's private airfield, during steep turn back to airport descended into trees. |
| 7/31/01 | St. Mary's, GA | 360 | IMC | Fatal (2) | Maneuvering | Loss of control in IMC | Student and CFI on IFR flight plan at 6000' talking to ATC, dodging heavier build-ups, entered a squall line off-shore, radar showed rapidly descending turn followed by crash into ocean. |
| 8/17/01 | Placerville, CA | 350 | VMC | Fatal (2) | Maneuvering | Low Pass | Pilot made a low pass over his home then crashed in neighbor's yard for unknown reasons. Pilot was a 15,000 hr. ATP. |
| 12/5/01 | Tecumseh, MI | 235 | VMC | None | Take-off | Crosswind | Taking off in gusting crosswind, wing contacted runway and pilot lost control. |
| 5/16/02 | Hattiesburg, MS | Super ES | VMC | Fatal (2) | Take-off | Power Loss | During a touch-and-go, witnesses heard a popping sound, saw aircraft in steep right turn, following 3 more rapid pops aircraft nosed down into wooded area and exploded. Pilot had traces of marijuana in blood and urine. |
| 7/13/02 | Sedona, AZ | 320 | VMC | None | Landing | Crosswind | During crosswind landing left wing contacted ground, followed by prop and nose. |
| 8/15/02 | Iola, KS | 320 | IMC | Minor | Forced landing | Power Loss | On instrument approach, lost power, restart unsuccessful, landed in farm field, hit berm and nosed over. |
| 10/15/02 | Coeur d'Alene, ID | IV | VMC | Serious (1) Fatal (2) | Landing | Landing | Very experienced Lancair pilot unfamiliar with private 2200' x 40' runway with trees on either side, flew unstable final, touched down with right main off pavement and veered into trees. |
| 11/5/02 | Bozeman, MT | IV-P | VMC | Fatal (1) | Approach | Power Loss | On 5-mile visual approach after 3-hour-plus cross-country, pilot reported losing power, witness reported a steep right turn followed by steep dive into a field just short of a housing development. Fuel valve was found to be between detents. |
| 11/21/02 | Belverde, TX | IV-P | VMC | None | Landing | Landing | During third attempt to land aircraft departed runway under heavy braking 2,300’ down a 3,000' runway, struck runway lights, ditch & fence. |
| 12/20/02 | Geelong, AUS | IV | VMC | Fatal (2) | Maneuvering | Stall/spin | Fifth flight since completion, performing stall sequences, failed to recover from stall/spin. |
| 12/28/02 | Payson, AZ | 235 | VMC | Fatal (1) | Aborted landing | No/Low Lancair Time | Pilot had just bought plane, no Lancair training, first flight, first and second landing attempts aborted, stall/spin on third abort. |
| 12/30/02 | Corona, CA | 320 | VMC | None | Forced landing | Engine | Lost oil pressure in cruise, forced landing on freeway, oil line had come loose, oil pumped overboard. |
| 4/26/03 | Wamego, KS | IV | VMC | None | Final | Landing | S-turning on short final for spacing behind slower aircraft, inadvertently contacted runway while turning. |
| 5/16/03 | Allendale, SC | IV | IMC | Fatal (4) | Cruise | Loss of control in IMC | IFR cross-country, maneuvering to avoid thunderstorms, broke up in flight. |
| 7/26/03 | Page, AZ | 235 | VMC | None | Landing | Landing | On landing aircraft porpoised and landed hard on nose gear, which collapsed. Aircraft skidded on nose, fuel line broke and fire started. |
| 7/27/03 | North Las Vegas, NV | IV-P | VMC | Minor | Take-off | Take-off | After being warned of possible wake turbulence from preceding aircraft, pilot attempted take-off and lost control, landing hard and collapsing gear.. |
| 7/30/03 | Fort Payne, AL | 235 | VMC | Minor | Landing | Engine | After local flight of unreported duration, engine quit on final, crashed into trees. |
| 8/1/03 | Oshkosh, WI | 360 | VMC | None | Landing | Landing | During EAA convention, RV had just landed on right side of runway, Lancair was landing on left side of runway per ATC instructions, RV turned left in front of Lancair and the two collided. |
| 8/7/03 | Vancouver, WA | 360 | VMC | None | Forced landing | Engine | Climbing out after take-off pilot noticed a puff of smoke from cowling, elected to return to airport, reduced power to descend and did 180, then added power to level off for traffic pattern, engine didn’t respond, landed on nearby road and struck rear of garbage truck, aircraft burned after pilot exited. |
| 9/2/03 | Manassas, VA | IV-P | VMC | None | Take-off | Take-off | Pilot distracted by copying complex IFR clearance during pre-take-off check, forgot to extend flaps to take-off position, during rotation aircraft settled back on runway, drifted off the right side of the runway and struck a sign, came to rest upright in grass. |
| 9/19/03 | Kimberling City, MO | Legacy | VMC | None | Take-off | Take-off | Pilot had placed temporary paper cover on pitot tube with no flag. Forgot to remove prior to flight. During attempted take-off from 2,600 ft. runway with trees at departure end, noticed abnormally low airspeed, continued to accelerate and rotate, got aural AOA warning, decided to abort, could not stop before hitting trees. |
| 10/20/03 | Knoxville, TN | 360 | VMC | Serious (1) Fatal (1) | Forced landing | Loss of Power | Immediately after take-off engine began to run rough, as pilot maneuvered to return to airport, engine quit, plane crashed into rocky embankment a few yards short of runway. |
| 11/10/03 | Memphis, TN | 235 | VMC | None | Forced landing | Loss of Power | Immediately after take-off engine lost power, pilot landed on highway adjacent to airport. |
| 2/17/04 | Mesa, AZ | IV-P-T | VMC | Fatal (2) | Maneuvering | Stall/Spin | Turbine IV-P on third test flight. Pilot witness saw aircraft in a flat spin before impact. |
| 4/7/04 | Novalato, Mexico | IV-P | VMC | Fatal (3) | Maneuvering | Stall/Spin | Mexican owner purchased American-built IV-P, receiving qualification instruction from an American CFI using an interpreter, witnesses reported stall/spin. |
| 4/23/04 | Memphis, TN | Legacy | IMC | Fatal | Cruise | Loss of control in IMC | Pilot was being vectored by ATC to avoid thunderstorms. ATC issued a recommended heading through a band of cells which pilot followed, ended up flying into a Level 5 cell and losing control. Aircraft was seen in a descending right turn before disappearing from radar. Wreckage indicated aircraft was intact when it struck the ground. Editorial note: a person familiar with this airplane reported on the LML that the only attitude indicator this plane had was the Dynon D-10, for which, according to Dynon, “maximum rates of change in attitude are limited to 150 degrees/second in roll, pitch, and yaw. Aircraft maneuvers outside this envelope may result in an inaccurate display and a warning message is shown. The instrument will return to specified accuracy within 10 seconds after the aircraft returns to coordinated flight with rates of change in attitude of no more than 4 degrees/second.” |
| 5/15/04 | Grand Canyon, AZ | IV-P | VMC | Fatal (4) | Maneuvering | Stall/spin | Aircraft fell out of the sky during sight-seeing flight over Grand Canyon. Witnesses reported aircraft in spin prior to impact. Pilot and three adult pax. Possible over-gross/aft CG situation with accelerated stall during maneuvering. |
| 5/31/04 | Vermontville, MI | IV-P | IMC | Fatal (3) | Cruise | Loss of control in IMC | IFR cross-country, pilot reported unable to maintain altitude, witness reported seeing plane in flat spin just prior to impact. Radar review showed aircraft entering an area of Level 6 precip followed by rapid descent. ATC never advised pilot of the Level 5 & 6 cells. |
| 6/13/04 | Hays, KS | 235 | VMC | None | Landing | Crosswind | Departed runway on landing roll-out due to loss of control in 30 degree crosswind, 23 kts. gusting to 28 kts. |
| 7/12/04 | Pacoima, CA | 360 | VMC | None | Landing | Tail Wind | Pilot acknowledged a 10-knot tailwind warning, touched down 1,800’ down a 4,125’ runway, locked brakes on the last 400’ of runway, ran off end into fence. DUH! |
| 8/1/04 | Madison, WI | Legacy | VMC | Fatal (1) | Forced Landing | Engine failure | Supercharged Legacy leaving Oshkosh, pilot previously commented to others he had been seeing high engine temps, reported engine problems to ATC 10 minutes into flight at 12,000’, diverted to Dane Co. Regional Airport 10 miles away, failed to make airport, crashed and burned after hitting light pole during engine-out approach to a grassy strip adjacent to a street. Engine teardown revealed one piston burned through and other heat damage not related to post-crash fire. Pilot overflew another airport enroute to Dane Co. |
| 8/27/04 | Houston, TX | 320 | VMC | None | Take-off | Take-off | Pilot reported he over-rotated on take-off, aircraft settled back onto runway drifting left due to right quartering crosswind, struck edge lights, taxiway sign and culvert, nose gear collapsed. |
| 9/3/04 | Chewelah, WA | 320 | VMC | Minor (2) | Maneuvering | Engine failure | Pilot reported this was first flight after engine rebuild, orbited over airport at 75% power for 30 minutes, maneuvering for landing, lowered gear and smelled strong odor of fuel, engine sputtered and quit, couldn’t make runway, crash-landed in open field. |
| 9/4/04 | Port Townsend, WA | 360 | VMC | None | Landing | Landing Gear | On sixth touch-and-go, pilot forgot to lower gear. |
| 9/10/04 | Orange, TX | 360 | VMC | Fatal (1) | Maneuvering | Stall/spin | CFI witness reported seeing aircraft doing a low pass over airport with gear down (unknown reason), followed by pull up, and then what appeared to be a power-on stall, recovery, then high speed impact with terrain. |
| 9/12/04 | Cambridge, OH | Legacy | VMC | None | Maneuvering | Structural Failure | Upper skin of left stub wing departed aircraft while in a left turn, aircraft landed safely. Examination showed poor bonding between skin and wing structure. |
| 10/26/04 | Sylvester, GA | IV-P-T | VMC | None | Landing | Loss of Power | Turbine IV-P entering downwind for landing, pilot added power to arrest descent, engine did not respond, plane crash-landed short of runway and skidded into parked aircraft on ramp. |
| 11/20/04 | Las Cruces, NM | 360 | VMC | None | Take-off | Loss of Control | Pilot reported that as he rotated for take-off, the aircraft “turned right” and departed the runway. The left main and nose gear were sheared off and the left wing “bent” aft. No wind information provided. |
| 1/5/05 | Sacaton, AZ | 235 | VMC | Fatal (2) | Maneuvering | Stall/spin | Witness reported watching the plane doing maneuvers when it “zoomed up” in a nose-high attitude, a wing dropped and the plane entered a spin from which it did not recover prior to impact. |
| 1/12/05 | El Cajon, CA | 235 | VMC | Serious (1) Fatal (1) | Landing | CFIT/O | Aircraft on short final for landing struck the airport perimeter fence 500’ short of the runway and impacted the ground. A passenger survived. She said the pilot did not make any comments about the approach that indicated any concern. |
| 4/8/05 | Bend, OR | Legacy | VMC | None | Landing | Landing Gear | Pilot forgot to lower gear for landing. |
| 5/1/05 | Nacodoches, TX | Legacy | VMC | None | Maneuvering | Loss of Power | While maneuvering at 700-800’ agl, engine lost power. Pilot “traded airspeed for altitude” and attempted to regain power, but was unable. Pilot attempted a landing in an open field. As the plane touched down the main gear sheered off and the plane skidded to a stop. A post-crash fire consumed the plane after the pilot and passenger exited the plane. Plane had accumulated 30 hours since completion. |
| 8/9/05 | Sundre, Alberta, Canada | IV-P-T | IMC | Fatal (2) | Cruise | Loss of Control in IMC | IFR cross-country, flew into an area of strong storms, broke up in flight. |
| 8/14/05 | Groveland, CA | Legacy | VMC | None | Cruise | Fuel Exhaustion | Pilot believed he had 18 gal. fuel on board upon departure for a trip he calculated would require 8 gal. Engine quit 6 miles short of destination, pilot made gear-up landing in an open field. It was determined the fuel tank sensors were improperly calibrated. |
| 8/15/05 | Madras, OR | Legacy FG | VMC | Fatal (2) | Maneuvering | Stall/spin | Witnesses reported hearing a sputtering engine and seeing the plane in a spin prior to impact. |
| 10/14/05 | Rootstown, OH | 235 | VMC | Fatal (4) | Cruise | Mid-air | Lancair on short cross-country collided with a Cessna 172 on an instrument instructional flight. Pilot and pilot-rated passenger in Lancair and private pilot instrument student and CFII in Cessna died in resulting crashes. |
| 11/27/05 | Martha’s Vineyard, MA | 360 | VMC | Fatal (1) | Take-off | Take-off | On initial climb-out after taking off for second test flight after completion, pilot reported a problem and attempted to return for landing, crashed into trees 200 feet outside airport fence. |
| 12/13/05 | Medina, OH | 235 | VMC | None | Landing | Landing | Landing on runway with snow and ice patches, pilot lost directional control and impacted banked snow on side of runway. Pilot had 11 hours in the aircraft. |
| 12/24/05 | Ramona, CA | 320 | VMC | Minor (2) | Forced Landing | Loss of Power | Pilot had just purchased aircraft 2 days prior to the accident. Returning from 1 hour cross-country, descending from cruise altitude of 8,500’ to airport landing pattern (2,400’), pilot pulled throttle back to 15” and prop to 2300. At approx. 6,000’ and 14 miles from airport, pilot configured aircraft for landing, including going to full rich mixture while continuing descent. Passing through 4,000’, noticed MAP and RPM decreasing, pilot made quick attempt to recover power, then looked for emergency landing area. Landed in a rocky field with gear down. Nose gear sheared and apparently ruptured a fuel line. Aircraft caught fire and was destroyed after pilot and passenger exited with minor injuries. |
| 1/28/06 | Santa Ynez, CA | 360 | VMC | Fatal (1) | Landing | Uncontrolled descent | Aircraft returning from local flight, pilot broadcast on CTAF he was “on the 45,” less than one minute later plume of smoke was observed 1 mile southwest of airport (rwy. 8-26). Aircraft had crashed in steep nose-down attitude. Pilot was one of the highest time LNC2 builder/pilots of the bunch. |
| 2/6/06 | Thermal, CA | 360 | VMC | None | Forced Landing | Engine Failure | While maneuvering at 2,000’ agl during a BFR, pilot stated “part of the engine came out through the cowl.” Pilot diverted to Thermal airport but was unable to make the field. Struck power lines while landing on a highway, hard landed. |
| 4/5/06 | Bankstown, AU | 360 | VMC | Fatal (1) | Take-off | Loss of Control | Aircraft was observed making a right turn following take-off, aircraft descended in turn and impacted ground. |
| 4/6/06 | Lakeland, FL | 320 | VMC | None | Landing | Landing | Aircraft hard-landed and broke off left main, veering into a ditch. |
| 4/24/06 | Charleston, SC | 360 | VMC | Fatal (1) | Take-off | Loss of Power | After third touch and go, aircraft was observed entering cross-wind turn, then was observed in a 90 degree left wing down roll, followed by the nose falling through and dive to the ground. There was some question as to whether the aircraft had been refueled after having been de-fueled for a new weight & balance. There was no fuel smell at the crash scene, and no fire. |
| 5/8/06 | San Diego, CA | IV-P-T | VMC | None | Landing | Landing | Gear was lowered on approach to landing with “3 green.” Immediately following, a total electrical power failure occurred. Pilot initiated a go-around and switched to back-up power. Gear lights now indicated left main not locked. Tower said there appeared to be 3 down. Upon touchdown, left main folded. Pilot stated he should have manually pumped the gear down to get it into locked position. |
| 5/10/06 | Somerset, PA | IV-P | VMC | Fatal (2) | Take-off | Fuel | Aircraft had just completed its 25-hour fly-off and was being ferried to its owner. Left-seat pilot was a 2,500 hr. commercial pilot with 500 hrs. in LNC4s and was the lead instructor in the Lancair-approved training program for Lancair pilots. Right-seat pilot was 5,600 hr. commercial pilot. Witnesses reported extensive ground checks and run-ups by the crew prior to take-off. Upon take-off, aircraft was trailing heavy black smoke and engine began sputtering. At tree-top altitude aircraft entered a high angle-of-attack left yaw, then dove to the ground. EFIS data showed fuel flow during take-off 20% above maximum, which during simulation after the accident, triggered an “ENGINE” warning light on the EFIS panel. It was observed that the high/low boost switch was positioned next to the “Nav” light switch and could be easily activated when activating the nav lights. Engine teardown revealed all plugs dark and sooty. NTSB probable cause: The pilot's failure to maintain sufficient airspeed after takeoff to preclude a stall, which resulted in a loss of control. Factors associated with the accident are the inadvertent stall, the loss of engine power due to the pilot's inadvertent activation of the high pressure auxiliary fuel pump, and the pilot's failure to abort the takeoff after receiving abnormal engine and fuel flow warnings on the primary flight display. |
| 6/2/06 | Montgomery, NY | 360 | IMC | Fatal (1) | Approach | Loss of control or CFIT | Aircraft on ILS approach, impacted wooded terrain 4 miles from airport. |
| 6/8/06 | Provo, UT | IV-P-T | Night VMC | Fatal (3) | Landing | CFIT | Aircraft was on a night VMC/VFR approach over water to Rwy. 13. Aircraft was on runway heading 750’ above TDZ 1.5 miles from threshold at 210 kts. Per EFIS, pilot adjusted prop to a high drag setting and began a descending right turn. Aircraft began to slow while descending. Last data point showed aircraft in right turn heading 322 degrees, airspeed decreasing through 123 kts, altitude 4,567’ and descent rate of 1,300 fpm. Elevation of the lake surface is 4,489’. Aircraft impacted water. |
| 8/4/06 | Omaha, NE | IV-P | VMC | None | Forced landing | Loss of Power | Aircraft suffered loss of power during take-off and during forced landing collided with a taxiing Piper Cherokee. |
| 8/22/06 | Helena, MT | IV-P-T | VMC | Minor | Forced landing | Engine failure | During touch and go’s pilot over-torqued and over-temped the turbine, causing catastrophic engine failure, force landed in a field. Pilot stated he should have installed torque-limiter. |
| 10/22/06 | Scappoose, OR | 320 | VMC | None | Landing | Loss of control | During local flight, aircraft lost electrical power. Pilot returned to home airport and followed an aircraft on approach. Due to that aircraft’s delay in exiting runway, pilot added power for go around and, by his own admission, failed to adequately counter torque with rudder, allowing the aircraft roll left, causing left wing to contact the ground and cart-wheel the aircraft. |
| 11/6/06 | Dalton, GA | IV-P-T | VMC | Fatal (1) | Forced landing | Loss of power | Aircraft enroute on an instrument flight plan, pilot advised ATC he was descending without engine power and requested vector to nearest airport. 5 miles out pilot reported he could not make airport and would land on a road. Witness reported aircraft in a steep right bank trying line up with the road. Right wing-tip contacted road and aircraft cart-wheeled in. |
| 1/23/07 | Guntersville, AL | 360 | VMC | None | Taxi | Engine fire | After landing pilot smelled burning fiberglass while taxiing in, then observed smoke and fire coming from the cowl. Aircraft was shutdown and pilot egressed. Aircraft was substantially damaged. Builder had used plastic automotive fuel hose in plumbing the fuel system. |
| 4/2/07 | Los Angeles, CA | 320 | VMC | None | Take-off | Loss of power | Aircraft impacted airport perimeter fence after a rejected take-off. Pilot reported a loss of power during take-off. Number 2 injector was found to be fouled. |
| 4/9/07 | Scottsbluff, NE | 200 | VMC | Fatal (2) | Maneuvering | Stall/spin | Pilot was giving local high school students rides in connection with their aerodynamics studies. Aircraft was observed maneuvering at approx. 1,000’ agl with varying engine sounds. Nose of aircraft dropped sharply and aircraft spun one and a half times to the right before impact. |
| 4/14/07 | Fulton, NY | Legacy | VMC | Fatal (2) | Take-off | Fuel | Aircraft’s first flight after completion. Immediately after take-off, witnesses reported engine noise “oscillating” and sputtering. Pilot was unable to maintain altitude and aircraft impacted a wooded area adjacent to the airport. Engine controls were found in the full forward position. There was no fire, but there was fuel odor present (both wing tanks were compromised in the crash). Inspection revealed no fuel in any fuel lines forward of the firewall. The FBO confirmed the aircraft had been fueled prior to the flight. Another Lancair owner at the airport reported he had been present when the builder/pilot of the accident aircraft attempted the first ground run of the engine, and there were difficulties in getting it running. There was a fuel spill of several gallons on the floor of the cockpit and inside the left wing root, emanating from a fitting in the fuel return line. The witness was not aware of how the leak was repaired. The multi-function engine display was sent to its manufacturer but no data could be recovered. |
| 5/2/07 | McAllen, TX | IV-P-T | VMC | Fatal (2) | Forced landing | Loss of power | Aircraft experienced a hot start prior to departure for this flight. After consultation with an A&P, pilot restarted aircraft and took off. Pilot reported to ATC a loss of power one minute into climb-out. Pilot attempted to land on a highway and struck concrete barriers in a construction zone. Aircraft was substantially damaged and was destroyed by a post-crash fire. |
| 6/2/07 | Parowan, UT | IV-P-T | VMC | None | Forced landing | Engine failure | Aircraft cruising at 26,000’ when engine failed, pilot saw smoke emanating from the exhaust. Pilot performed an emergency descent to 12,500’ and attempted restart, which was unsuccessful. Pilot diverted to nearest airport and performed a power-off approach and landing. Aircraft ran off the end of the runway and gear collapsed. Pilot stated he had been in and out of clouds during cruise. Engine inlet was not equipped with anti-ice. |
| 6/15/07 | Portland, OR | IV-P-T | VMC | Fatal (3) | Departure | Uncontrolled descent | Aircraft was climbing on course 6 minutes after take-off when radio and radar contact was lost. Last radar hit showed aircraft in a descending right turn. Witnesses reported aircraft exited cloud base at high speed in near vertical descent. Aircraft crashed in the back yard of a residence 12 miles from the departure airport. |
| 7/20/07 | Destin, FL | 320 | VMC | None | Forced landing | Prop failure | Outboard 10” of one prop blade departed in flight, pilot secured engine after violent shaking made control difficult, diverted to Destin airport in glide but was unable to make runway, hard landed. |
| 8/1/07 | Sylvan, KS | IV | VMC | Serious | Forced landing | Engine failure | Crash landing in farm field after engine failure due to oil starvation due to unsecured oil line to the wastegate actuator |
| 9/11/07 | Alabaster, AL | IV-P | VMC | Minor | Landing | Landing | Touched down 8 feet short of runway threshold, right main torn off by pavement edge, aircraft departed right side of runway and impacted ditch and trees. |
| 10/13/07 | Great Harbor Cay. Bahamas | IV | IMC | Fatal (1) | Cruise | UNK | Aircraft encountered “severe, severe turbulence” per pilot, entered descending right turn from 20,000 feet to 14,000 feet, then disappeared from radar. Aircraft pieces and a life raft with the plane’s N-number were found in the water by the Coast Guard near the last radar hit. |
| 2/08/08 | Albany, OR | ES | VMC | Fatal (3) | Cruise Climb | Stall/spin | Aircraft climbing to cruise altitude of 13,000 feet after 1010 departure from Albany Municipal. Passing through 10,400 feet at 1017 pilot declared an emergency. Radar contact lost at 1018. Witness saw aircraft come spinning out of the cloud base at approx. 2,000 feet. Witness reported engine revving up and down during spin. |
| 2/16/08 | Portland, OR | Legacy FG | IMC | Fatal | Approach | Uncontrolled descent | After missing an ILS approach on first attempt, aircraft veered off course during second attempt and impacted a road 4500 feet from the approach end of the runway. Wx reported as wind calm, viz ¼ in fog, clouds broken 100 feet, broken 900 feet. The report did not mention whether the pilot had an instrument rating. |
| 2/26/08 | Temecula, CA | IV-P | VMC | Serious (2) Minor (2) | Forced landing | Engine failure | Pilot reported engine “just quit, no sputtering or surging” as he turned to final for landing. After switching tanks and turning on boost pump pilot could not restart. He selected a clear area and landed off airport. Landing gear collapsed on ground contact at 70 knots. 25 gallons of fuel were found remaining in the tanks. |
| 4/2/08 | Portland, OR | 320 | VMC | None | Landing | Electrical failure | Aircraft lost electrical power in flight, pilot returned to airport, gear was dropped per LNC2 procedures, gear collapsed 500-600 feet after touch-down. |
| 4/13/08 | Lakeland, FL | Legacy | VMC | Fatal (1) | Departure | Uncontrolled descent | Witnesses reported pilot struggling to secure canopy before take-off, a photo posted to LML showed the canopy open after take-off, witness reported a reduction in power, followed by the nose and left wing falling through with the plane disappearing behind a tree line, followed by a plume of smoke. |
| 4/23/08 | Mesa, AZ | IV-P | VMC | Fatal (3) | Take-off | Uncontrolled descent | On take-off as gear was retracting, tower controller advised the pilot that the aircraft was trailing smoke, no response from the pilot, aircraft made an abrupt climbing left turn, stalled, descended straight down into an orange grove and exploded. There were rumors of possible mis-fueling, but it was not mentioned in the NTSB report. |
| 5/8/08 | Snow Hill, North Carolina | IV-P | IMC | Fatal (2) | Cruise | Uncontrolled descent | Aircraft encountered unexpected icing, pilot requested descent, aircraft disappeared from radar soon after clearance to descend, aircraft impacted ground in typical flat spin attitude. |
| 5/30/08 | Murrieta, CA | Legacy FG | VMC | Fatal (2) | Forced landing | Fuel | Aircraft doing touch-and-go’s, on downwind engine surged and quit, aircraft stalled and nosed in during aggressive turn toward runway. No evidence of fuel in or around aircraft after crash. |
| 6/6/08 | Alhambra, IL | IV-P | IMC | Fatal (1) | Cruise | Loss of Control in IMC | Pilot maneuvering to get around areas of heavy precipitation using Stormscope and ATC advisories. Pilot advised ATC he was “turning around,” radar contact was lost, aircraft experienced in-flight break-up with wings being found 4,100 feet from fuselage. |
| 6/8/08 | Bend, OR | Legacy | VMC | None | Forced landing | Engine failure | During flight at 9,000 feet, engine oversped then quit, on dead-stick approach to Bend airport, touched down short of runway, sheered off main gear and slid down runway. Examination revealed a 4 inch diameter hole in the top of the case above the #3 cylinder. |
| 6/18/08 | Springfield, KY | IV-P | VMC | Fatal (1) | Forced landing | Engine failure | Pilot reported loss of oil pressure, crashed on approach to airport. |
| 7/7/08 | Hollywood, FL | IV-P | VMC | Fatal (1) | Take-off | Engine | After take-off at about 50 feet agl engine power was “interrupted,” aircraft began steep left turn, at 80 degrees bank angle nose fell through, aircraft impacted in steep nose down attitude. |
| 7/31/08 | Oshkosh, WI | Legacy FG | VMC | Fatal (2) | Landing | Stall | Aircraft on short approach behind slower traffic, witnesses observed nose fall through followed by power up and roll to the right, aircraft impacted ground inverted. |
| 8/1/08 | Yakima, WA | ES | IMC | Fatal (2) | Cruise | Loss of control in IMC | Aircraft descended into cloud tops, possibly due to mountain wave activity, entered right spiral descent, broke up in flight.. |
| 8/11/08 | Stafford, VA | IV-P | VMC | None | Cruise | Electrical fire | After take-off, pilot turned on stand-by battery switch to charge battery, immediately smelled and saw smoke filling the cockpit. Returned for immediate landing and discovered fire in the aft battery compartment. Determined regulators had not been properly mounted. |
| 8/25/08 | Vancouver, WA | 360 | VMC | None | Landing | Landing | On short final at 20 feet agl aircraft suddenly sank and hard-landed, collapsing main gear. |
| 9/2/08 | Camarillo, CA | Propjet | VMC | Minor | Landing | Engine failure | During cruise climb immediately after take-off pilot noticed loss of power and white smoke from the exhaust stacks, performed 180 and set up for a power-off approach to runway. Due to overrun barriers on the approach end of the runway, pilot maintained higher than normal approach speed and altitude, ran off the far end of the runway and nosed over. |
| 10/18/08 | Parowan, UT | Legacy | VMC | Fatal (1) Serious (1) | Take-off | Loss of control | Witnesses reported items flying out of the cockpit immediately after take-off, aircraft leveled off at about 400’ agl, entered gentle descending left turn until wingtip contacted ground and aircraft cart-wheeled in. Post-accident investigation indicated canopy had not been latched. Autopsy of pilot indicated two painkillers present that would typically cause impairment. |
| 11/2/08 | Socorro, NM | Legacy | VMC | Minor (1) | Landing | Landing | High, fast approach, aircraft floated considerable distance down runway, ran off end and was substantially damaged. |
| 12/29/0/ | Owensboro, KY | 360 | VMC | None | Forced landing | Fuel | At some point during flight while switching tanks, the cotter pin in the fuel valve handle sheered but the pilot did not realize it, ran the tank dry and could not switch to full tank. Force landed into a field |
| 1/1/09 | Joliet, IL | 360 | VMC | Fatal (2) | Take-off | Uncontrolled descent | After a refueling stop on cross-country, aircraft stalled on take-off, impacting left wing down. Calculations showed aircraft to be slightly over gross and CG slightly aft of aft limit. Another pilot who departed about 2 hours before the accident aircraft reported that he had a moderate build-up of frost on his wings, which he wiped off before take-off. Accident aircraft took off after dark. |
| 1/7/09 | Iola, KS | IV | VMC | Minor (1) | Aborted Landing | Loss of power | On short final pilot observed aircraft landing opposite direction, pilot applied full power for go-around, as aircraft began to climb engine experienced complete loss of power. Aircraft hard-landed in the grass beside the runway and caught fire. |
| 1/30/09 | Longmont, CO | Legacy | VMC | Minor (1) | Take-off | Loss of control | Upon take-off pilot observed that canopy was not latched. Thinking he could not stop on remaining runway, he continued take-off and the canopy began oscillating up and down, causing significant pitch oscillations. Pilot flew pattern for a landing experimenting with speed and power settings to minimize pitch oscillations. Pilot stated aircraft was barely controllable on short final, impacting the ground short of runway. |
| 3/28/09 | Milton, FL | 320 | VMC | Minor (1) | Landing | Crosswind | Landing on Runway 18 with winds reported 210 at 10 kts. gusting 25, 10 ft. above runway wind gust kicked right wing up 30 degrees, left wing struck ground left of runway, aircraft cart-wheeled in. |
| 5/22/09 | Seligman, AZ | IV | VMC | Fatal (2) | UNK | UNK | Aircraft on cross-country from Hillsboro, OR to Scottsdale, AZ, 3 hours into flight, ATC lost contact, wreckage found the next day. |
| 7/1/09 | Santa Clarita, CA | IV | VMC | Fatal (2) | Maneuvering | Uncontrolled descent | Witnesses reported aircraft doing aerobatics at about 2000 ft. agl. Aircraft did a roll then a loop, during which the aircraft stalled and descended to the ground. |
| 9/5/09 | Louisa, VA | 235 | VMC | None | Taxiing | Collision | Tail-dragger Cessna taxied into the back of the 235, damaging horizontal stabilizer and empennage. |
| 11/21/09 | Watsonville, CA | Legacy | VMC | Fatal (1) | Cruise | Uncontrolled descent | Pilot declared emergency at 10,000 ft. stating he had lost engine power and was going to try to make Watsonville Airport. Pilot then stated he had a fire, was at 5,000 ft. and had the airport in sight. Aircraft crashed ¼ mi. short of the runway. Investigation determined engine had lost all oil due to a broken oil return line from the separator. Pilot had done all maintenance on the aircraft himself with no A&P or Repairman certificate (report did not indicate if he built the plane). |
| 11/29/09 | Torrance, CA | 235 | VMC | None | Taxiing | Collision | Tail-dragger Travel Air taxied into tail of stationary Lancair as Lancair was preparing for taxi in front of his hangar. |
| 3/15/10 | Hilton Head, SC | IV-P | VMC | Fatal (1) | Forced landing | Collision with pedestrian | Aircraft lost propeller while in cruise flight, made power-off landing on beach with wind screen covered in oil and no forward visibility, struck a pedestrian on the beach. Prop separated due to fatigue cracks in the crank flange due to an apparently unrecorded prop strike. |
| 6/25/10 | Burns, OR | Legacy | VMC | None | Take-off | Crosswind | Taking off on Runway 30, upon lift-off aircraft rolled right and wing contacted runway and aircraft impacted ground. A routine aviation weather report (METAR) for the departure airport, issued at the time of the accident, reported winds from 170 degrees at 20 knots, gusting to 32 knots with squall conditions. DUH! |
| 6/27/10 | Caldwell, ID | IV | VMC | None | Start-up | Engine fire | Upon start-up, pilot notice odor like burning plastic, within 30 seconds he observed smoke emanating from the engine area. He shut down the engine and notice flames at the bottom of the cowl. Investigation showed a fitting on a hydraulic pump had backed off and sprayed hydraulic fluid on the right exhaust manifold. The annual condition inspection had been performed on the aircraft 2 flight hours previously. |
| 7/27/10 | Spokane, WA | 360 | VMC | Minor (1) | Landing | Loss of control | Pilot was experiencing electrical problems during approach and landing and was unsure gear was down. While maneuvering for landing the aircraft went into an uncommanded left roll which the pilot could only control by reducing power. Aircraft stalled on short final and impacted short of the runway. Damage to landing gear indicated partial extension. |
| 9/2/10 | McMinnville, TN | 320 | VMC | Minor (1) | Take-off | Loss of control | Pilot took off with canopy only partially latched. While attempting to latch canopy pilot allowed aircraft to descend and impact ground. |
| 11/29/10 | Hubbard, OR | 360 | VMC | Fatal (1) | Take-off | Uncontrolled descent | Upon take-off aircraft entered a slow roll to the left, reaching 90* angle of bank, aircraft descended into terrain. Gear remained extended during entire flight. Autopsy revealed levels of cough-suppressant and antihistamines that would have been impairing. Pilot did not have valid medical certificate, as his last application (9/11/08) had been denied due to heart problems and post-traumatic stress syndrome.. |
| 12/25/10 | Troy, MI | Legacy | VMC | None | Landing | Loss of control | Prior to flight pilot received WX briefing with no reports of icing enroute. During flight aircraft picked up 6-10 mm of ice in spite of an “operable” de-ice system. Pilot maintained higher than normal airspeed on approach, but over the threshold experienced an uncommanded roll to the left, with the left wing striking the runway and the aircraft cart-wheeling. |
| 2/13/11 | Dawsonville, GA | 320 | VMC | None | Maneuvering | Loss of power | While in maneuvering flight engine began to lose power. Pilot had just transferred fuel from wing tanks to header tank 10 minutes previously. Pilot went through restart procedures but was unable to recover power. Force-landed on a roadway with substantial damage to left wing, prop and rudder. Upon FAA inspection, aircraft was restarted on ground with no anomalies. |
| 2/26/11 | Milford, UT | 360 | IMC | Fatal (1) | Cruise | Loss of control in IMC | VFR on-top on VFR flight plan, pilot reported to ATC having engine problems, requested vector to nearest airport. Pilot (not instrument rated) soon reported having trouble controlling the airplane, but about a minute later reported being stabilized. ATC asked the pilot if he was IMC, but as pilot began to respond, transmission was cut-off and ATC radar indicated rapid descent, and then aircraft disappeared from radar. Wreckage was found near the last radar hit. |
| 4/3/11 | Chesapeake, VA | 360 | VMC | Fatal (2) | Take-off | Uncontrolled descent | After take-off at about 200’ agl engine began “sputtering” or “back-firing,” aircraft entered a steep right turn and began losing altitude, after about 90* of turn, nose dropped, aircraft rolled inverted and impacted ground. Pilot witness said aircraft entered a “classic stall/spin.” |
| 4/8/11 | Temple Bar, AZ | 320 | VMC | Minor (1) | Cruise | Loss of power | During cruise flight engine began to gradually lose power. Pilot performed engine recovery procedures but continued to lose power, reaching “idle” at about 400’ agl. Aircraft substantially damaged in forced landing. |
| 4/15/11 | Baltimore, MD | 320 | VMC | None | Landing | Landing | After high and fast approach, upon touch-down aircraft bounced twice and veered hard left before coming to a stop. Nose gear separated, left main forced up through wing, right main folded. Pilot stated he should have gone around after first bounce. |
| 5/2/11 | Kansas City, MO | Legacy | VMC | None | Landing | Landing | Pilot had been cleared to land on runway 1L, touched down in the grass area between parallel taxiways A and B and struck a sign. Pilot stated he should have aborted poor approach and should have familiarized himself with the airport layout before arrival. |
| 5/6/11 | Richlands, NC | Legacy | VMC | Fatal (2) | Approach | UNK | During descent from cruise for approach to landing in VMC, pilot declared emergency due to low fuel pressure. ATC asked for fuel remaining and pilot reported one hour. ATC gave vector to airport and pilot reported airport in sight. No further contact. Wreckage was found 3 miles from airport. |
| 5/18/11 | Pioneer, CA | Legacy | VMC | Fatal (1) | UNK | UNK | Aircraft reported missing during flight from Minden, NV to Tracy, CA. Wreckage located in mountainous terrain at 6,500’. VMC conditions prevailed for time and route of flight. |
| 5/21/11 | Albuquerque, NM | 320 | VMC | None | Landing | Crosswind | Landing to the southwest with a 16 kts. cross wind, wind suddenly shifted to the north and gusted to 36 kts per ATCT. Pilot attempted to abort landing but touched down, bounced three times and yawed until nose and right main folded. Rain squalls were in the vicinity of the airport. |
| 6/12/11 | Azle, TX | 360 | VMC | Serious (1) | Cruise | Loss of power | Aircraft recently completed, on a local test flight. During cruise engine began losing power. Pilot unsuccessfully attempted power recovery, engine quit on final to a forced landing to a field. Aircraft substantially damaged on hard landing. No info on cause of loss of power. |
| 6/30/11 | Yukon, OK | 320 | VMC | Fatal (1) | Approach | Uncontrolled descent | 7 mile repositioning flight, witnesses at departure airport reported engine sounding like it was running rough on take-off. At arrival airport witnesses heard rough-running aircraft approaching airport. As aircraft turned from base to final, bank angle increased to nearly 90*, nose fell through and aircraft spiraled to the ground. |
| 7/3/11 | Port O’Connor, TX | IV-P | VMC | Fatal (2) | Take-off | Uncontrolled descent | Climbing out after take-off, aircraft made a “sharp” 180 and began descending as if returning to the airport. As the wings rolled level, aircraft made a “snap roll” and spun into the water. |
| 8/20/11 | Hammonton, NJ | IV-P | VMC | Fatal (1) | Maneuvering | Mid-air Collision | Yak doing aerobatics in an NOTAMed but uncontrolled aerobatic box, Lancair cruising at 3,500’, aircraft collided and both crashed. Yak pilot parachuted and survived. Witnesses said neither aircraft took evasive action. |
| 8/25/11 | Heath, Ohio | 235 | VMC | Fatal (1) | Take-off | Loss of control | Pilot’s first flight since purchasing aircraft from builder. Witnesses reported on take-off aircraft was very unstable with large oscillations in both pitch and roll. Upon reaching approximately 150’, aircraft banked sharply to the left and descended into trees. Witnesses said engine sounded normal through entire flight. |