| Sabot Legacy Condition Checklist |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ITEM |  | CRITERIA |  | DISCREPENCY AND NOTES |  | OK |  | LUBE |  | SPECIFI-CATION |
| Registration Number: N26XY |  |  |  |  |  |  |  |  |  |  |  |
| Date: ______________ |  |  |  |  |  |  |  |  |  |  |  |
| Total time in service: ________ |  |  |  |  |  |  |  |  |  |  |  |
| Time since last inspection: ________ |  |  |  |  |  |  |  |  |  |  |  |
| Inspector: __________________________ |  |  |  |  |  |  |  |  |  |  |  |
| PREPARATION |  |  |  |  |  |  |  |  |  |  |  |
| Fly airplane for ½ to 1 hour. Test brakes, test gyros, radios, navigation equipment, all systems. |  |  |  |  |  |  |  |  |  |  |  |
| While engine is still warm, drain oil, replace oil filter, pull top spark plugs, perform compression test, pull bottom spark plugs, clean and gap spark plugs, reinstall spark plugs. |  |  |  |  |  |  |  |  |  |  |  |
| Remove all inspection plates, Cowling, aft bulkhead, baggage area floor boards, seat bottoms. |  |  |  |  |  |  |  |  |  |  |  |
| Remove ELT. Replace battery |  |  |  |  |  |  |  |  |  |  |  |
| CABIN SECTION |  |  |  |  |  |  |  |  |  |  |  |
|  | SYSTEMS CHECKS DURING FLIGHT |  |  |  |  |  |  |  |  |  |  |
|  | Fuel Pump |  | Record fuel pressure in Lo Hi Record current in Lo  Hi  |  |  |  |  |  |  |  |  |
|  | Brakes |  | Smooth, no pump up |  |  |  |  |  |  |  |  |
|  | Chelton EFIS 1 |  | Check all Functions |  |  |  |  |  |  |  |  |
|  | Chelton EFIS 2 |  | Check all Functions |  |  |  |  |  |  |  |  |
|  | MVP-50 |  | Check against fuel level gauges |  |  |  |  |  |  |  |  |
|  | Altimeter |  | Check |  |  |  |  |  |  |  |  |
|  | Airspeed Indicator |  | Check |  |  |  |  |  |  |  |  |
|  | Communications transceiver #1 |  | Radio check 5x5 |  |  |  |  |  |  |  |  |
|  | Communications transceiver #2 |  | Radio check 5x5 |  |  |  |  |  |  |  |  |
|  | VOR Receiver |  | Radial track, verify OBS – CDI operation |  |  |  |  |  |  |  |  |
|  | VOR Receiver |  | Accuracy check |  |  |  |  |  |  |  |  |
|  | VOR Receiver |  | ILS/GS operation |  |  |  |  |  |  |  |  |
|  | GPS Receiver |  | Operation, reception |  |  |  |  |  |  |  |  |
|  | GPS Receiver |  | Data base up to date |  |  |  |  |  |  |  |  |
|  | Transponder |  | Check with ATC |  |  |  |  |  |  |  |  |
|  | Fuel Pressure on Left Tank |  | Record  |  |  |  |  |  |  |  |  |
|  | Fuel flow on Left tank |  | Record  |  |  |  |  |  |  |  |  |
|  | Fuel Pressure on Right Tank |  | Record  |  |  |  |  |  |  |  |  |
|  | Fuel flow on Right tank |  | Record  |  |  |  |  |  |  |  |  |
|  | Fuel Flow gauges |  | Check calibration against tank filling |  |  |  |  |  |  |  |  |
|  | Gear position lights |  | Extinguished when gear up, Illuminated when gear down |  |  |  |  |  |  |  |  |
|  | Dimmers |  | Lights dim with each knob. Knobs secure |  |  |  |  |  |  |  |  |
|  | Landing gear up operation |  | <5-sec retraction |  |  |  |  |  |  |  |  |
|  | Landing gear down operation |  | <5-sec extension |  |  |  |  |  |  |  |  |
|  | Emergency operation |  | <10-sec extension |  |  |  |  |  |  |  |  |
|  | Hydraulic pump |  | No cycling when up or down |  |  |  |  |  |  |  |  |
|  | Hydraulic pump |  | Shuts off after gear is up and down, Log current due to hydraulic pump on Primary bus  |  |  |  |  |  |  |  |  |
|  | Magnetic Compass |  | Check northerly heading  |  |  |  |  |  |  |  |  |
|  | Magnetic Compass |  | Check easterly heading  |  |  |  |  |  |  |  |  |
|  | Magnetic Compass |  | Check westerly heading  |  |  |  |  |  |  |  |  |
|  | Magnetic Compass |  | Check southerly heading  |  |  |  |  |  |  |  |  |
|  | Engine Shutdown with fuel selector |  | Time from selecting OFF until engine stops  |  |  |  |  |  |  |  |  |
| FUSELAGE GENERAL CONDITION |  |  |  |  |  |  |  |  |  |  |  |
|  | AIRWORTHINESS CERTIFICATE |  |  |  |  |  |  |  |  |  |  |
|  | In aircraft |  | Displayed |  |  |  |  |  |  |  |  |
|  | REGISTRATION |  |  |  |  |  |  |  |  |  |  |
|  | In aircraft |  | Displayed |  |  |  |  |  |  |  |  |
|  | OWNERS MANUAL (POH) |  |  |  |  |  |  |  |  |  |  |
|  | In aircraft |  | Up to date |  |  |  |  |  |  |  |  |
|  | WEIGHT & BALANCE |  |  |  |  |  |  |  |  |  |  |
|  | In POH |  | Complete, up to date, signed, matches equipment list |  |  |  |  |  |  |  |  |
|  | In Log book |  | Complete, up to date, signed, matches equipment list |  |  |  |  |  |  |  |  |
|  | STATE REGISTRATION |  |  |  |  |  |  |  |  |  |  |
|  | Displayed on right side of fuselage |  | Current |  |  |  |  |  |  |  |  |
|  | DATA PLATE |  |  |  |  |  |  |  |  |  |  |
|  | Displayed on left side of fuselage |  | Secure and legible |  |  |  |  |  |  |  |  |
|  | PLACARDS |  |  |  |  |  |  |  |  |  |  |
|  | Registration Number Placard |  | Installed on left instrument panel |  |  |  |  |  |  |  |  |
|  | Passenger Warning |  | Installed on right instrument panel |  |  |  |  |  |  |  |  |
|  | Gear Switch Placard |  | Installed near gear switch |  |  |  |  |  |  |  |  |
|  | Circuit breaker placards |  | Installed above breakers |  |  |  |  |  |  |  |  |
|  | Experimental placard |  | Installed on inside of canopy |  |  |  |  |  |  |  |  |
|  | Canopy Latch |  | Down indicator function check |  |  |  |  |  |  |  |  |
|  | Canopy Latch |  | Lcoked indicator function check |  |  |  |  |  |  |  |  |
|  | Cabin |  |  |  |  |  |  |  |  |  |  |
|  | Console box |  | Clean out |  |  |  |  |  |  |  |  |
|  | Console box flashlight |  | Replace batteries |  |  |  |  |  |  |  |  |
|  | Carbon Monoxide Indicator |  | check |  |  |  |  |  |  |  |  |
|  | Seat Belts |  | Securely bolted |  |  |  |  |  |  |  |  |
|  | Shoulder straps |  | Securely bolted |  |  |  |  |  |  |  |  |
|  | Upholstery condition |  | No interference with controls |  |  |  |  |  |  |  |  |
|  | Fuel Pump Drain |  | Clear, no drips |  |  |  |  |  |  |  |  |
|  | Fuel Pump |  | Secure, no leaks |  |  |  |  |  |  |  |  |
|  | Ground power connector |  | Secure connections |  |  |  |  |  |  |  |  |
|  | Flap Actuator |  | Wires free of chafe |  |  |  |  |  |  |  |  |
|  | Flap Actuator |  | Relays secure |  |  |  |  |  |  |  |  |
| WING |  |  |  |  |  |  |  |  |  |  |  |
|  | Bolts |  | Installed & Torqued |  |  |  |  |  |  |  |  |
| BATTERIES |  |  |  |  |  |  |  |  |  |  |  |
|  | Primary Battery Connections |  | Secure, safety wired |  |  |  |  |  |  |  |  |
|  | Battery #1 |  | Corrosion-free |  |  |  |  |  | Wipe down with baking soda (if lead acid) |  |  |
|  | Battery Electrical condition. Load test, with landing lights and taxi lights. |  | Voltage >12vdc |  |  |  |  |  |  |  |  |
| RUDDER PEDALS |  |  |  |  |  |  |  |  |  |  |  |
|  | Operation |  | Smooth |  |  |  |  |  |  |  |  |
|  | Control cable connections |  | Secure thimbles |  |  |  |  |  |  |  |  |
|  | Attachment bolts |  | Secure |  |  |  |  |  |  |  |  |
|  | Travel |  | Equal left to right |  |  |  |  |  |  |  | Balance weight clears vert. Stab by 1” |
| BRAKES |  |  |  |  |  |  |  |  |  |  |  |
|  | Travel clearances |  | No interference |  |  |  |  |  |  |  |  |
|  | Secure fittings |  |  |  |  |  |  |  |  |  |  |
|  | Uniform pressure |  |  |  |  |  |  |  |  |  |  |
|  | Reservoir |  | Full |  |  |  |  |  | MIL-H-5606 |  |  |
| ENGINE INSTRUMENTS |  |  |  |  |  |  |  |  |  |  |  |
|  | Correct & Working |  |  |  |  |  |  |  |  |  |  |
|  | Security Behind Panel |  |  |  |  |  |  |  |  |  |  |
|  | Range Card |  | On-board |  |  |  |  |  |  |  |  |
| ENGINE CONTROLS |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Lever |  | Full & Cushion |  |  |  |  |  |  |  |  |
|  | Power Lever Idle Stop |  | Hard stop, idle throttle stop |  |  |  |  |  |  |  |  |
|  | Power Lever Friction Lock |  | Set |  |  |  |  |  |  |  |  |
| FLIGHT INSTRUMENTS |  |  |  |  |  |  |  |  |  |  |  |
|  | Compass |  | Swing |  |  |  |  |  |  |  |  |
|  | Compass Card |  | Installed on compass |  |  |  |  |  |  |  |  |
|  | Airspeed Calibration Card |  | In POH |  |  |  |  |  |  |  |  |
|  | Pitot/Static Lines |  | Clean |  |  |  |  |  |  |  |  |
|  | Static System |  | Certification |  |  |  |  |  |  |  |  |
|  | Behind Panel |  | Secure |  |  |  |  |  |  |  |  |
| AVIONICS |  |  |  |  |  |  |  |  |  |  |  |
|  | Correct & Working |  |  |  |  |  |  |  |  |  |  |
|  | Transponder |  | Certification |  |  |  |  |  |  |  |  |
|  | Behind Panel |  | Secure |  |  |  |  |  |  |  |  |
|  | Connectors |  | Secure |  |  |  |  |  |  |  |  |
| FUEL SYSTEM |  |  |  |  |  |  |  |  |  |  |  |
|  | Lines Secure |  |  |  |  |  |  |  |  |  |  |
|  | Fuel Valve Function |  |  |  |  |  |  |  |  |  |  |
|  | Left/Right/Off |  |  |  |  |  |  |  |  |  |  |
|  | Labels |  |  |  |  |  |  |  |  |  |  |
|  | Fuel Filter |  |  |  |  |  |  |  |  |  |  |
| FLIGHT CONTROLS |  |  |  |  |  |  |  |  |  |  |  |
|  | Secure |  | Deflection at trailing edges < ½ trailing edge thickness  |  |  |  |  |  |  |  |  |
|  | Free |  | No binding or friction |  |  |  |  |  |  |  |  |
|  | Full Travel |  | Stop to stop |  |  |  |  |  |  |  |  |
|  | Control Travel |  | Correct direction |  |  |  |  |  |  |  |  |
|  | Aileron Control Stops |  | Tight |  |  |  |  |  |  |  |  |
|  | Control Stick |  | Labels |  |  |  |  |  |  |  |  |
|  | Walking Beam |  |  |  |  |  |  |  |  |  |  |
|  | Jam Nuts |  | Tight |  |  |  |  |  |  |  |  |
|  | Elevator Pivot bolts |  | Secure |  |  |  |  |  |  |  |  |
|  | Elevator pushrod |  | Secure |  |  |  |  |  |  |  |  |
| ELECTRICAL SYSTEM |  |  |  |  |  |  |  |  |  |  |  |
|  | Position Lights |  | Operational, clean connections |  |  |  |  |  |  |  |  |
|  | Landing Lights |  | Operational |  |  |  |  |  |  |  |  |
|  | Taxi Lights |  | Operational |  |  |  |  |  |  |  |  |
|  | Instrument lights |  | Operational |  |  |  |  |  |  |  |  |
|  | Circuit Breakers |  | Operational |  |  |  |  |  |  |  |  |
|  | Radio Cables |  | Secure connection |  |  |  |  |  |  |  |  |
|  | Antennas |  | No damage |  |  |  |  |  |  |  |  |
|  | Strobe Lights |  | Operational |  |  |  |  |  |  |  |  |
|  | Pitot heat |  | Operational, Record current  |  |  |  |  |  |  |  |  |
|  | Head phone connections |  | Operational |  |  |  |  |  |  |  |  |
|  | Cabin light |  | Operational |  |  |  |  |  |  |  |  |
| ELT |  |  |  |  |  |  |  |  |  |  |  |
|  | Installation |  | Check clamp and connectors |  |  |  |  |  |  |  |  |
|  | Battery corrosion |  | Clean |  |  |  |  |  |  |  |  |
|  | Controls and sensor |  | Check operation |  |  |  |  |  |  |  |  |
|  | Operation |  | Reception on 121.5MHz at distance |  |  |  |  |  |  |  |  |
|  | Battery Date |  | Valid until next inspection |  |  |  |  |  |  |  |  |
| EMPENNAGE |  |  |  |  |  |  |  |  |  |  |  |
| VERTICAL STABILIZER & RUDDER |  |  |  |  |  |  |  |  |  |  |  |
|  | Rudder Attach Bolts |  | Secure, no play |  |  |  |  |  |  |  |  |
|  | Rudder Movement |  | Free & Correct |  |  |  |  |  |  |  | Equal travel |
|  | Push rod end bearing |  | Lubricate |  |  |  |  |  | Grease |  |  |
|  | Push rod end bearing bolt |  | Secure and tight |  |  |  |  |  |  |  |  |
|  | Balance weight |  | Secure |  |  |  |  |  |  |  |  |
| HORIZONTAL STABILIZER & ELEVATOR |  |  |  |  |  |  |  |  |  |  |  |
|  | Elevator Attach Bolts |  | Secure, no play |  |  |  |  |  |  |  |  |
|  | Elevator Movement |  | Free, Correct |  |  |  |  |  |  |  | +26, -11 |
|  | Balance Weight |  | Secure |  |  |  |  |  |  |  |  |
|  | Elevator Travel |  | Full |  |  |  |  |  |  |  |  |
|  | Push rod end bearing |  | Lubricate |  |  |  |  |  | Grease |  |  |
|  | Push rod end bearing bolt |  | Secure and tight |  |  |  |  |  |  |  |  |
| ELEVATOR TRIM SYSTEM |  |  |  |  |  |  |  |  |  |  |  |
|  | Secure |  |  |  |  |  |  |  |  |  |  |
|  | Correct & Functioning |  |  |  |  |  |  |  |  |  |  |
|  | Control Travel |  | Full |  |  |  |  |  |  |  |  |
|  | Wires |  | Free From Chafing |  |  |  |  |  |  |  |  |
|  | Balance Weight |  | Secure |  |  |  |  |  |  |  |  |
| WINGS |  |  |  |  |  |  |  |  |  |  |  |
| LEFT WING TIP |  |  |  |  |  |  |  |  |  |  |  |
|  | Fuel Vent |  | Free |  |  |  |  |  |  |  |  |
|  | Position Lights |  | Lenses secure, no cracks |  |  |  |  |  |  |  |  |
|  | Strobe Light |  | Lenses secure, no cracks |  |  |  |  |  |  |  |  |
|  | Wires |  | Free From Chafing |  |  |  |  |  |  |  |  |
| AILERON/LEFT |  |  |  |  |  |  |  |  |  |  |  |
|  | Attach Screws |  | Secure |  |  |  |  |  |  |  |  |
|  | Push rod clevis |  | Secure, no play |  |  |  |  |  |  |  |  |
|  | Neutral Adjustment |  | Center stick |  |  |  |  |  |  |  |  |
|  | Up Adjustment/Stop |  | Full |  |  |  |  |  |  |  | +___ |
|  | Down Adjustment/Stop |  | Full |  |  |  |  |  |  |  | -___ |
|  | Bellcrank Neutral Adjustment |  |  |  |  |  |  |  |  |  |  |
|  | Bolts, Lock Nuts |  | Secure |  |  |  |  |  |  |  |  |
|  | Pushrod Clearance |  | Free |  |  |  |  |  |  |  |  |
|  | Travel |  | Correct direction |  |  |  |  |  |  |  |  |
| AILERON TRIM SYSTEM |  |  |  |  |  |  |  |  |  |  |  |
|  | Aileron |  | Secure |  |  |  |  |  |  |  |  |
|  | Pilot Function |  | Correct, Operational |  |  |  |  |  |  |  |  |
|  | Copilot Function |  | Correct, Operational |  |  |  |  |  |  |  |  |
|  | Control Travel |  | Full |  |  |  |  |  |  |  |  |
|  | Wires |  | Free From Chafing |  |  |  |  |  |  |  |  |
| LEFT WING TIP |  |  |  |  |  |  |  |  |  |  |  |
|  | Fuel Vent |  | Free |  |  |  |  |  |  |  |  |
|  | Position Lights |  | Lenses secure, corrosion free |  |  |  |  |  |  |  |  |
|  | Strobe Light |  | Operational, lens secure |  |  |  |  |  |  |  |  |
|  | Wires |  | Free From Chafing |  |  |  |  |  |  |  |  |
| AILERON/RIGHT |  |  |  |  |  |  |  |  |  |  |  |
|  | Attach Bolts |  | Secure |  |  |  |  |  |  |  |  |
|  | Neutral Adjustment |  | Center stick |  |  |  |  |  |  |  |  |
|  | Up Adjustment/Stop |  | Full |  |  |  |  |  |  |  |  |
|  | Down Adjustment/Stop |  | Full |  |  |  |  |  |  |  |  |
|  | Bellcrank Neutral Adjustment |  |  |  |  |  |  |  |  |  |  |
|  | Bolts, Lock Nuts |  | Secure |  |  |  |  |  |  |  |  |
|  | Pushrod Clearance |  | Free |  |  |  |  |  |  |  |  |
|  | Travel |  | Correct distance and direction |  |  |  |  |  |  |  |  |
| FUEL TANK/LEFT |  |  |  |  |  |  |  |  |  |  |  |
|  | Fuel Lines |  | Secure |  |  |  |  |  |  |  |  |
|  | Fuel Lines |  | Free From Chafing |  |  |  |  |  |  |  |  |
|  | Vent Lines |  | Secure |  |  |  |  |  |  |  |  |
|  | Vent Lines |  | Free From Chafing |  |  |  |  |  |  |  |  |
|  | Wires Secure |  |  |  |  |  |  |  |  |  |  |
|  | Wires |  | Free From Chafing |  |  |  |  |  |  |  |  |
|  | Fuel filler cap O-ring |  | Replace |  |  |  |  |  |  |  |  |
|  | Fuel Type & Quantity |  | Indicated |  |  |  |  |  |  |  |  |
| FUEL TANK/RIGHT |  |  |  |  |  |  |  |  |  |  |  |
|  | Fuel Lines |  | Secure |  |  |  |  |  |  |  |  |
|  | Fuel Lines |  | Free From Chafing |  |  |  |  |  |  |  |  |
|  | Vent Lines |  | Secure |  |  |  |  |  |  |  |  |
|  | Vent Lines |  | Free From Chafing |  |  |  |  |  |  |  |  |
|  | Wires |  | Secure |  |  |  |  |  |  |  |  |
|  | Wires |  | Free From Chafing |  |  |  |  |  |  |  |  |
|  | Fuel filler cap O-ring |  | Replace |  |  |  |  |  |  |  |  |
|  | Fuel Type & Quantity |  | Indicated |  |  |  |  |  |  |  |  |
| PITOT/STATIC SYSTEM |  |  |  |  |  |  |  |  |  |  |  |
|  | Mast |  | Secure |  |  |  |  |  |  |  |  |
|  | Wiring |  | Secure |  |  |  |  |  |  |  |  |
|  | Wires |  | Free From Chafing |  |  |  |  |  |  |  |  |
|  | Pitot Line |  | Secure |  |  |  |  |  |  |  |  |
|  | Static Line |  | Secure |  |  |  |  |  |  |  |  |
|  | Static System |  | Tested |  |  |  |  |  |  |  |  |
| AUTO PILOT |  |  |  |  |  |  |  |  |  |  |  |
|  | Pitch Servo |  | Secure |  |  |  |  |  |  |  |  |
|  | Roll Servo |  | Secure |  |  |  |  |  |  |  |  |
|  | Pitch linkage |  | Free |  |  |  |  |  |  |  |  |
|  | Roll linkage |  | Free |  |  |  |  |  |  |  |  |
|  | Wires |  | Secure |  |  |  |  |  |  |  |  |
|  | Wires |  | Free From Chafing |  |  |  |  |  |  |  |  |
|  | Operation |  | Correct direction |  |  |  |  |  |  |  |  |
|  | CWS and disconnect |  | Correct operation |  |  |  |  |  |  |  |  |
| FLAPS |  |  |  |  |  |  |  |  |  |  |  |
|  | Actuator |  | Correct & Functioning |  |  |  |  |  |  |  |  |
|  | Travel Marks |  | Installed |  |  |  |  |  |  |  |  |
|  | Travel & Uniformity |  | Smooth, no binding |  |  |  |  |  |  |  |  |
|  | Jam Nuts |  | Secure |  |  |  |  |  |  |  |  |
|  | Travel Stops Up/Down |  | Limit switches stop motion at limits |  |  |  |  |  |  |  |  |
| NOSE WHEEL |  |  |  |  |  |  |  |  |  |  |  |
|  | Tire inflation |  | 35 to 45 psig |  |  |  |  |  |  |  |  |
|  | Strut inflation |  | At least ¾” compression |  |  |  |  |  |  |  |  |
|  | Strut condition |  | No leakage, no corrosion |  |  |  |  |  |  |  |  |
|  | Bearing (lift nose wheel off ground) |  | No play, free spin |  |  |  |  |  |  |  |  |
|  | Overcenter link |  | No pivot wear |  |  |  |  |  |  |  |  |
|  | Position switch |  | Operational |  |  |  |  |  |  |  |  |
|  | Bearings |  | Grease Packed |  |  |  |  |  |  |  |  |
|  | Wheel fork |  | Secure, Bolts torqued |  |  |  |  |  |  |  |  |
|  | Hydraulic Lines |  | Secure, No chafe, no cracking |  |  |  |  |  |  |  |  |
|  | Tire Condition |  | Tread depth, no cracks |  |  |  |  |  |  |  |  |
| MAIN LANDING GEAR |  |  |  |  |  |  |  |  |  |  |  |
|  | Strut inflation |  | At least 2.5” extended |  |  |  |  |  |  |  |  |
|  | Strut condition |  | No leakage, no corrosion |  |  |  |  |  |  |  |  |
|  | Freedom Of Movement |  | Grease Packed |  |  |  |  |  |  |  |  |
|  | Bearings |  | No play, free spin |  |  |  |  |  |  |  |  |
|  | Brake Linings |  | New |  |  |  |  |  |  |  |  |
|  | Brake Discs |  | No score, no glaze |  |  |  |  |  |  |  |  |
|  | Hydraulic Lines |  | Secure, No chafe, no cracking |  |  |  |  |  |  |  |  |
|  | Hydraulic Fluid Quantity |  | Full, no leakage |  |  |  |  |  |  |  |  |
|  | Overcenter link |  | No pivot wear |  |  |  |  |  |  |  |  |
|  | Position switch |  | Operational |  |  |  |  |  |  |  |  |
|  | Brake lines |  | No leaks, no chafe, secure |  |  |  |  |  |  |  |  |
|  | Wheel Fork |  | Secure, Bolts torqued |  |  |  |  |  |  |  |  |
|  | Tire Inflation |  | 45 to 55psig |  |  |  |  |  |  |  |  |
|  | Tire Condition |  | Tread depth, no cracks |  |  |  |  |  |  |  |  |
|  | Inner Gear Doors |  | Secure, no linkage play |  |  |  |  |  |  |  |  |
|  | Outer Gear Doors |  | Secure fittings |  |  |  |  |  |  |  |  |
|  | LANDING GEAR OPERATION – On Jacks |  |  |  |  |  |  |  |  |  |  |
|  | Normal operation |  | Proper operation, doors close tightly |  |  |  |  |  |  |  |  |
|  | Hydraulic system |  | No leaks |  |  |  |  |  |  |  |  |
|  | Air springs |  | Force overcenter links |  |  |  |  |  |  |  |  |
|  | Emergency extension operation |  | Gear switch shuts off pump, valve operates |  |  |  |  |  |  |  |  |
|  | Wheel well to tire clearance |  |  |  |  |  |  |  |  |  |  |
|  | Brake pads |  | Replace |  |  |  |  |  |  |  |  |
|  | Wheel bearings |  | Clean and pack |  |  |  |  |  | Grease |  |  |
| ENGINE SECTION |  |  |  |  |  |  |  |  |  |  |  |
|  | Engine Mount Bolts |  | Tight |  |  |  |  |  |  |  |  |
|  | Engine Vibration Pads/Bolts |  | Tight |  |  |  |  |  |  |  |  |
|  | Engine Grounds |  | Clean and tight |  |  |  |  |  |  |  |  |
|  | Spark Plug Wires |  | Secure, no chafe |  |  |  |  |  |  |  |  |
|  | Cylinder Head Temperature Sensors Wires |  | Correct & Secure |  |  |  |  |  |  |  |  |
|  | Alternator |  | Secure, no oil leaks, tight connections |  |  |  |  |  |  |  |  |
|  | Starter Security |  | Tight |  |  |  |  |  |  |  |  |
|  | Starter Electrical Connections |  | Clean and tight |  |  |  |  |  |  |  |  |
|  | Plugs |  | Clean & Gap |  |  |  |  |  |  |  |  |
|  | Engine Oil |  | Change |  |  |  |  |  | Exxon Elite 15W-50 |  |  |
|  | Engine Oil Filter Security |  | Replace, cut and inspect |  |  |  |  |  | Champion CH48108 |  |  |
|  | EGT Sensors |  | No erosion |  |  |  |  |  |  |  |  |
|  | Air Filter |  | Clean, Re-oil |  |  |  |  |  |  |  |  |
|  | Cylinder Compression |  |  |  |  |  |  |  |  |  |  |
|  | 1_____ | 2_____ |  | 3_____ |  | 4 _____ |  | 5_____ |  | 6 _____ |  |
|  | Fuel Lines |  | Secure, no leaks, no chafe, no cracks |  |  |  |  |  |  |  |  |
|  | Injectors |  | Secure, no leaks |  |  |  |  |  |  |  |  |
|  | Induction Hoses & Clamps |  | Tight |  |  |  |  |  |  |  |  |
|  | Fuel Distribution Block |  | Fittings tight, no leaks, filter clean and flow free |  |  |  |  |  |  |  |  |
|  | Cylinder Lines |  | Secure, no leaks, no chafe, no cracks |  |  |  |  |  |  |  |  |
|  | Throttle Connection |  | Secure |  |  |  |  |  |  |  |  |
|  | Throttle stroke |  | Stop to stop |  |  |  |  |  |  |  |  |
|  | Air Filter |  | Secure |  |  |  |  |  |  |  |  |
|  | Fuel Filter |  | Clean and flow free |  |  |  |  |  |  |  |  |
| Cooling System |  |  |  |  |  |  |  |  |  |  |  |
|  | Baffles & Seals |  | Good condition and flexibility |  |  |  |  |  |  |  |  |
|  | Heat Muff & Shrouds |  | No cracks |  |  |  |  |  |  |  |  |
|  | Exhaust Gaskets |  | No leaks |  |  |  |  |  |  |  |  |
|  | Exhaust Security/Nuts |  | Tight |  |  |  |  |  |  |  |  |
|  | Exhaust Hangers |  | Tight, no cracking |  |  |  |  |  |  |  |  |
| Propeller |  |  |  |  |  |  |  |  |  |  |  |
|  | Prop Bolts |  | Torqued 50 ft-lbs |  |  |  |  |  |  |  |  |
|  | Propeller Blade |  | No damage, dents, dings, erosion |  |  |  |  |  |  |  |  |
|  | Spinner |  | No cracks, secure |  |  |  |  |  |  |  |  |
|  | Spinner Back plates |  | Secure, nut plates secure |  |  |  |  |  |  |  |  |
|  | Propeller |  | Blade track |  |  |  |  |  |  |  |  |
|  | Propeller hub |  | No leaks |  |  |  |  |  |  |  |  |
| Cowl |  |  |  |  |  |  |  |  |  |  |  |
|  | Check Fit |  |  |  |  |  |  |  |  |  |  |
|  | Check Security |  |  |  |  |  |  |  |  |  |  |
|  | Check Seals/Induction |  |  |  |  |  |  |  |  |  |  |
|  | Check Seals/Cooling |  |  |  |  |  |  |  |  |  |  |
|  | Check For Wear & Cracking |  |  |  |  |  |  |  |  |  |  |
|  | Check For Heat Damage |  |  |  |  |  |  |  |  |  |  |
|  | Check Exhaust Clearance |  |  |  |  |  |  |  |  |  |  |
| Operational Check Engine |  |  |  |  |  |  |  |  |  |  |  |
|  | Oil Pressure |  |  |  |  |  |  |  |  |  |  |
|  | Magneto Check |  | Drop Mag ____ |  |  |  |  |  |  |  |  |
|  | Lightspeed Ign Check |  | Drop Electronic Ign ____ |  |  |  |  |  |  |  |  |
|  | Static RPM _____ |  |  |  |  |  |  |  |  |  |  |
|  | Max MAP _____ |  |  |  |  |  |  |  |  |  |  |
|  | Alternator #1 |  | Voltage > 13.8V |  |  |  |  |  |  |  |  |
|  | Alternator #2 |  | Voltage > 13.8V |  |  |  |  |  |  |  |  |
|  | Boost Off Fuel Pressure |  |  |  |  |  |  |  |  |  |  |
|  | Boost On Fuel Pressure |  |  |  |  |  |  |  |  |  |  |
|  | Inspect For Leaks |  |  |  |  |  |  |  |  |  |  |
|  | N26XY Systems Checklist |  | Completed |  |  |  |  |  |  |  |  |
|  | AD Review |  |  |  |  |  |  |  |  |  |  |
|  | Mfg. Bulletins Review |  |  |  |  |  |  |  |  |  |  |
|  | SEAT & SEAT BELTS |  |  |  |  |  |  |  |  |  |  |
|  | Seat Belts |  | Correct & Secure |  |  |  |  |  |  |  |  |
|  | Shoulder Harness |  | Correct & Secure |  |  |  |  |  |  |  |  |
|  | Seat Bottoms |  | Secure |  |  |  |  |  |  |  |  |
|  | Recovery |  |  |  |  |  |  |  |  |  |  |
|  | Airplane interior |  | Vacuum all spaces |  |  |  |  |  |  |  |  |
|  | Airplane interior |  | Remove all tools and FOD |  |  |  |  |  |  |  |  |
|  | Inspection panels |  | Remove all FOD, install |  |  |  |  |  |  |  |  |
|  | Clean engine |  |  |  |  |  |  |  | Stoddard |  |  |
|  | Canopy |  | Clean canopy |  |  |  |  |  | Plastic window cleaner |  |  |
|  | Wash and wax airplane |  |  |  |  |  |  |  |  |  |  |
|  | Wheel wells |  | Clean |  |  |  |  |  |  |  |  |